Metrix AX1320 & AX2320 DC POWER SUPPLY

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Metrix AX1320 & AX2320 DC POWER SUPPLY.

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For GPIB instrument drivers:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

For VXI instrument drivers:

-  the instrument is installed in the VXI mainframe and you are

using one of the following controller options:

Embedded controller

AT-MXI

MC-MXI

GPIB-VXI

-  the logical address supplied to the initialize function must

match the logical address of the instrument.

-  if you are using GPIB-VXI, there must be at least one unused

device name in the handler.

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error Codes:

Error codes are returned as the return value of each instrument driver

function. A program should examine this value after each call to an instrument

driver function to determine if any error occurred. Possible error codes and

their meanings are listed with the corresponding instrument driver function.

________________________________________________________________________________

How To Use This Document:

This document is intended to be used as a programming reference manual.

It describes each function in the

Metrix AX1320 & AX2320 DC POWER SUPPLY

instrument. The functions are listed in alphabetical order with a

description of the function, C syntax of the function, a description of each

parameter, and possible error codes.

________________________________________________________________________________

Function Tree Layout:

Metrix AX1320 & AX2320 DC POWER SUPPLY Function Name:

Initialize                                  ax2320_init

Commands

Set Mode                                 ax2320_set_mode

Set Voltage (VSET)                       ax2320_set_voltage

Set Current (ISET)                       ax2320_set_current

Set Overvoltage (OVSET)                  ax2320_set_over_voltage

Queries

Read Voltage                             ax2320_read_voltage

Read Current                             ax2320_read_current

Read Overvoltage                         ax2320_read_over_voltage

Utility Functions

Write To Instrument                      ax2320_write

Read Instrument Data                     ax2320_read

Reset                                    ax2320_reset

Error Query                              ax2320_error_query

Revision Query                           ax2320_revision_query

Close                                       ax2320_close

________________________________________________________________________________

Metrix AX1320 & AX2320 DC POWER SUPPLY

Metrix AX1320/AX2320

PROGRAMMABLE DC POWER SUPPLY

This instrument module provides GPIB support for

the AX1320 and AX2320 power supplies.

The Programmable Power Supply Series from Metrix

offers a complete solution to power supply system

requirements.Theses models are indispensable

instruments in assisting in the development and

test of new products as well as being standart

instruments for automatic test systems. The wide

range of output selection combined with excellent

load/line regulation creates a vital instrument in

your lab environment.

AX3220H         Dual 32 volt, 2 amp, Quad mode

The module contains the following functions:

1. Initialise.

2. Configuration

1. Set Mode

2. Set Output

3. Master Output control

4. Status Reporting

3. Measurement and Status.

1. Read Output

2. Read Status

4.Local Storage.

1. Store

2. Recall

5. Close

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

ax2320_close

short ax2320_close (short IDInstrument);

Purpose

Panel name:     Close

Description:    Takes the instrument off-line.

The  instrument must  be

reinitialized to use it again.

Parameter List

IDInstrument

Variable Type       short

Control Name    : ID instrument

Description     : Accepts the instrument ID

returned by the Initialize

fonction to select the desired

instrument.

Valid Range     : 1 to Maximum Instance of this

instrument.

The Maximum Instances of this

instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system.The value is set

internally in the driver and is

typically 10.

Variable Type   : Integer

Return Value

Control name    : Error

Description     : This return value is placed in

the global variable ax2320_err.

Variable type   : Integer

Valid range     :

221     Unable to close instrument

________________________________________________________________________________

ax2320_error_query

short ax2320_error_query (short instrumentID, short *errorCode,

char errorMessage[]);

Purpose

Reads an error code from the instrument error queue.

Parameter List

instrumentID

Variable Type       short

Accepts the Instrument ID returned by the Initialize function to

select the desired instrument.

Valid Range:

1 to  Maximum Instances of this Instrument

The Maximum Instances of this Instrument is the maximum number of

instruments that can be controlled by this driver in a single system.

The value is set internally in the driver. The typical value is 10.

Default Value: 1

errorCode

Variable Type       short (passed by reference)

This is the error code read from the instrument's error queue.

errorMessage

Variable Type       char []

This is the error message string read from the instrument's error

message queue.

Return Value

Displays the results of the function call.

Error Codes:

0   Success

-1   Instrument ID out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

________________________________________________________________________________

ax2320_init

short ax2320_init (short GPIBAddress, short IDQuery, short resetDevice,

short *IDInstrument);

Purpose

Panel name  :   Initialise

Description :   Initialises the AX1320 or 2320

- opens the instrument and sets the GPIB

address to the specified value.

- Sets timeout 10s

- if ID Query On , identifies the model of

the instrument and sets the global

variable (ax2320_model).

- if Reset On , sends device clear and

sets the Power supply in the following

way :

CH1     OFF

CH2     OFF (AX2320)

TRK     OFF (AX2320)

OCP     OFF

VSET    0.000

ISET    0.000

OVSET   35

Parameter List

GPIBAddress

Variable Type       short

Control Name:  GPIB Address

Description:   Specifies the address on the GPIB

to be used to communicate

with the instrument.  This value

must correspond to the current

internal setting of the

instrument.

Variable Type: Integer

Valid Range:   0 to 30

Default Value: 12

NOTE:  The Default Value of this control should

be set to the factory default of the

instrument, however if your instrument's

actual setting is different this control's

IDQuery

Variable Type       short

Control Name:  ID Query

Description:   Specifies if an ID Query is sent

to the instrument during the

initialization procedure.

Variable Type: Integer

Valid Range:   0 - Skip Query

1 - Do Query (Default)

NOTE:  Under normal circumstances the ID Query

ensures that the instrument initialized

over the GPIB is the type supported by

this driver.  However circumstances may

arise where it is undesirable to send an

ID Query to the instrument.  In those

cases; set this control to "Skip Query"

and this function will initialize the

GPIB and the Command arrays in the driver,

without doing an ID Query.

resetDevice

Variable Type       short

Control Name:  Reset Device

Description:   Specifies if the instrument is to

be reset to its power-on settings

during the initialization

procedure.

Variable Type: Integer

Valid Range:   0 - Don't Reset

1 - Reset Device (Default)

NOTE: If you do not want the instrument reset.

Set this control to "Don't Reset" while

IDInstrument

Variable Type       short (passed by reference)

Control name    : ID Instrument

Description     : Returns an instrument ID that

is used in all subsequent

function calls to select the

instrument at the Address

specified.

If more than one instrument of

the same type is used,this value

will be used to differentiate

beetween them. The first

instrument initialized will be

assigned an Instrument ID of 1.

Variable Type   : Integer

Return Value

Control name    : Error

Description     : This return value is placed in

the global variable ax2320_err.

Variable type   : Integer

Valid range     :

0     Success

-1     GPIB Address out of range

-2     Identification out of range

-3     Reset out of range

220     Unable to open instrument

223     Instrument identification query failed

224     Error clearing instrument

230     Error writing to instrument

231     Error reading from instrument

233     Error in configuring GPIB address

239     Error in configuring timeout

________________________________________________________________________________

ax2320_read

short ax2320_read (short instrumentID, short numberBytesToRead,

char readBuffer[], short *numBytesRead);

Purpose

Panel Name:    Read Instrument Data

Description:   Reads data from instrument buffer

and returns it to the specified

variable in memory.

NOTES: (1) Because the instrument may return both

numeric and text data in response to

queries this function returns the data

in string format.

(2) If valid data is NOT available at the

instrument's output buffer when this

function is called the instrument will

hang up and the function will not

return until it times out.  If the

time-out is disabled this function

will hang indefinitely and it may be

neccessary to reboot the computer to

break out.

Parameter List

instrumentID

Variable Type       short

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Default Value: 1

numberBytesToRead

Variable Type       short

Control Name:  Number of Bytes to Read

Description:   The number of bytes specified by

this control should be greater

than or equal to the number of

bytes which are to be read from

the instrument. If the actual

number of bytes to be read is

greater than the number this

control specifies then multiple

reads will be necessary to empty

the instrument's output buffer.

If the instrument's output buffer

is not emptied the instrument may

return invalid data when future

reads are performed.

Variable Type: Integer

Default Value: 50 (See NOTE 2)

NOTES: (1) If NO DATA is available at the

instrument's output buffer when this

function is called the instrument will

hang up and the function will not

return until it times out.  If the

time-out is disabled, this function

will hang indefinitely and it may be

necessary to reboot the computer.

(2) If the number of bytes expected is

greater than 50 the value of this

control may be increased. If the

actual number of bytes read is larger

than the declared size of the read

buffer a run-time error will be

generated.

readBuffer

Variable Type       char []

Control Name:  Read Buffer

Description:   The incoming data from the

instrument is placed into this

variable.

Variable Type: String (char *)

NOTES: (1) Because the Instrument may return both

numeric and text data in response to

queries this function returns the data

in string form.

(2) This function does not overwrite any

old data left in the string variable

from the last time the function was

called. String data in LabWindows is

terminated with an ASCII null(Hex 0)

and string manipulation functions will

only recognize data before the ASCII

null.

(3) The declared size of the string

variable must be greater than the

actual number of bytes read from the

instrument if it is not a run-time

error will be generated.

numBytesRead

Variable Type       short (passed by reference)

Control Name:  Number of Bytes Read

Description:   This variable contains the actual

number of bytes read from the

instrument. This is the value

which is returned by ibrd in the

global variable ibcnt.

Variable Type: Integer *

NOTE: If the actual number of bytes read is less

than the number of bytes specified in

number of bytes to read then the output

buffer is probably empty. If the number of

bytes read is 0 and the instrument returns

a 231 error code the read function failed.

The most probable cause for the read to

fail is because there was no data available

at the instrument's output buffer.

Return Value

Control Name:  Error Code

Description:   Displays the results of the

function call.

Variable Type: Integer

Error Codes:

0   Success

-1   Instrument ID out of range

231   Error reading from instrument

232   Instrument not initialized

________________________________________________________________________________

ax2320_read_current

short ax2320_read_current (short IDInstrument, short channel, short type,

double *current);

Purpose

Panel name      :   Read Current

Description     :   Returns the programmed value

or the current output of the

channel.

Parameter List

IDInstrument

Variable Type       short

Control Name    : ID instrument

Description     : Accepts the instrument ID

returned by the Initialize

fonction to select the desired

instrument.

Valid Range     : 1 to Maximum Instance of this

instrument.

The Maximum Instances of this

instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system.The value is set

internally in the driver and is

typically 10.

Variable Type   : Integer

channel

Variable Type       short

Control name    :  Channel

Description     :  Indicates the channel to

which the overvoltage will

be read.

Variable type   :  integer

Valid range     :  1   CH 1

2   CH 2 (AX2320)

Default value   :  1   CH 1

type

Variable Type       short

Control name    :  Type

Description     :  Indicates if you read the

programmed value (ISET?) or the

output value (IOUT?).

Variable type   :  integer

Valid range     :  0   ISET?

1   IOUT?

Default value   :  0   ISET?

current

Variable Type       double (passed by reference)

Control name    :  Current

Description     :  Displays the current value from

the instrument (Scientist Nota-

tion).

Variable type   :  Double

Return Value

Control name    : Error

Description     : This return value is placed in

the global variable ax2320_err.

Variable type   : Integer

Valid range     :

0     Success

-1     ID Instrument out of range

-2     Channel out of range

-3     Type out of range

230     Error writing to instrument

231     Error reading from instrument

300     CH2 not available with AX1320 model.

________________________________________________________________________________

ax2320_read_over_voltage

short ax2320_read_over_voltage (short IDInstrument, short channel,

double *overvoltage);

Purpose

Panel name      :   Read Overvoltage

Description     :   Returns the programmed value.

Parameter List

IDInstrument

Variable Type       short

Control Name    : ID instrument

Description     : Accepts the instrument ID

returned by the Initialize

fonction to select the desired

instrument.

Valid Range     : 1 to Maximum Instance of this

instrument.

The Maximum Instances of this

instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system.The value is set

internally in the driver and is

typically 10.

Variable Type   : Integer

channel

Variable Type       short

Control name    :  Channel

Description     :  Indicates the channel to

which the overvoltage will

be read.

Variable type   :  integer

Valid range     :  1   CH 1

2   CH 2 (AX2320)

Default value   :  1   CH 1

overvoltage

Variable Type       double (passed by reference)

Control name    :  Overvoltage

Description     :  Displays the overvoltage value

from the instrument (Scientist

Notation).

Variable type   :  Double

Return Value

Control name    : Error

Description     : This return value is placed in

the global variable ax2320_err.

Variable type   : Integer

Valid range     :

0     Success

-1     ID Instrument out of range

-2     Channel out of range

230     Error writing to instrument

231     Error reading from instrument

300     CH2 not available with AX1320 model.

________________________________________________________________________________

ax2320_read_voltage

short ax2320_read_voltage (short IDInstrument, short channel, short type,

double *voltage);

Purpose

Panel name      :   Read Voltage

Description     :   Returns the programmed value

or the voltage output of the

channel.

Parameter List

IDInstrument

Variable Type       short

Control Name    : ID instrument

Description     : Accepts the instrument ID

returned by the Initialize

fonction to select the desired

instrument.

Valid Range     : 1 to Maximum Instance of this

instrument.

The Maximum Instances of this

instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system.The value is set

internally in the driver and is

typically 10.

Variable Type   : Integer

channel

Variable Type       short

Control name    :  Channel

Description     :  Indicates the channel to

which the voltage will be read

back.

Variable type   :  integer

Valid range     :  1   CH 1

2   CH 2 (AX2320)

Default value   :  1   CH 1

type

Variable Type       short

Control name    :  Type

Description     :  Indicates if you read the

programmed value (VSET?) or the

output value (VOUT?).

Variable type   :  integer

Valid range     :  0   VSET?

1   VOUT?

Default value   :  0   VSET?

voltage

Variable Type       double (passed by reference)

Control name    :  Voltage

Description     :  Displays the voltage value from

the instrument (Scientist Nota-

tion).

Variable type   :  Double

Return Value

Control name    : Error

Description     : This return value is placed in

the global variable ax2320_err.

Variable type   : Integer

Valid range     :

0     Success

-1     ID Instrument out of range

-2     Channel out of range

-3     Type out of range

230     Error writing to instrument

231     Error reading from instrument

300     CH2 not available with AX1320 model.

________________________________________________________________________________

ax2320_reset

short ax2320_reset (short instrumentID);

Purpose

Panel Name:    Reset

Description:   This function resets the

instrument to its default state.

Parameter List

instrumentID

Variable Type       short

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Default Value: 1

Return Value

Control Name:  Error Code

Description:   Displays status relating to the

function call.

Variable Type: Integer

Error Codes:

0   Success

-1   Instrument ID out of range

230   Error writing to instrument

232   Instrument not initialized

________________________________________________________________________________

ax2320_revision_query

short ax2320_revision_query (short instrumentID, char driverRevision[],

char firmwareRevision[]);

Purpose

Panel Name:    Revision Query

Description:   This function returns the revision

numbers of the instrument driver

and instrument firmware.

Parameter List

instrumentID

Variable Type       short

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Default Value: 1

driverRevision

Variable Type       char []

Control Name:  Driver Revision

Description:   This control returns the

Instrument Driver Revision.

Variable Type: String (char *)

firmwareRevision

Variable Type       char []

Control Name:  Instrument Firmware Revision

Description:   This control returns the

Instrument's Firmware Revision.

Variable Type: String (char *)

Return Value

Control Name:  Error Code

Description:   Displays status relating to the

function call.

Variable Type: Integer

Error Codes:

0   Success

-1   Instrument ID out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

________________________________________________________________________________

ax2320_set_current

short ax2320_set_current (short IDInstrument, short channel, double current);

Purpose

Panel name      :   Set Output Current

Description     :   Specify an output channel and

current in amps (i.e. no

milliamps): 0.000 to 2.000

If the output current exceeds

the maximum current rating of

the power supply when the OCP

is on, the unit will reduce

the output current to zero and

LDC will displays "OC".

For AX1320 , only CH1 is available.

Parameter List

IDInstrument

Variable Type       short

Control Name    : ID instrument

Description     : Accepts the instrument ID

returned by the Initialize

fonction to select the desired

instrument.

Valid Range     : 1 to Maximum Instance of this

instrument.

The Maximum Instances of this

instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system.The value is set

internally in the driver and is

typically 10.

Variable Type   : Integer

channel

Variable Type       short

Control name    :  Channel

Description     :  Indicates the channel to

which the current will be sent.

Variable type   :  integer

Valid range     :  1   CH 1

2   CH 2 (AX2320)

Default value   :  1   CH 1

current

Variable Type       double

Control name    : Current

Description     : Set the Current on CH1 or CH2

(AX2320). Values are in amps

(i.e. no milliamps).

If the output current exceeds

the maximum current rating of

the power supply when the OCP

is on, the unit will reduce

the output current to zero and

LDC will displays "OC".

Variable type   : Double

Valid range     : 0.000 to 2.000

Default value   : 1.000

Return Value

Control name    : Error

Description     : This return value is placed in

the global variable ax2320_err.

Variable type   : Integer

Valid range     :

0     Success

-1     ID Instrument out of range

-2     Channel out of range

-3     Current out of range

230     Error writing to instrument

231     Error reading from instrument

300     CH2 not available with AX1320 model.

________________________________________________________________________________

ax2320_set_mode

short ax2320_set_mode (short IDInstrument, short CH1, short CH2, short tracking,

short OCP);

Purpose

Panel name:     Set Mode

Description:    Set the following parameters

on or off.

Set Output CH1 on/off

Set Output CH2 on/off     (AX2320 only)

Set the tracking mode TRK (AX2320 only)

Set OverCurrent Protection on/off (OCP).

Parameter List

IDInstrument

Variable Type       short

Control Name    : ID instrument

Description     : Accepts the instrument ID

returned by the Initialize

fonction to select the desired

instrument.

Valid Range     : 1 to Maximum Instance of this

instrument.

The Maximum Instances of this

instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system.The value is set

internally in the driver and is

typically 10.

Variable Type   : Integer

CH1

Variable Type       short

Control name    :   CH1

Description     :   This command enables or

disables the output of CH1.

Variable type   :   Integer

Valid range     :   0 - CH1 Off

1 - CH1 On

Default value   :   1 - CH1 On

CH2

Variable Type       short

Control name    :   CH2

Description     :   This command enables or

disables the output of CH2.

This command is used for

AX2320 .

If Tracking mode is On , CH2

is identical to CH1.

Variable type   :   Integer

Valid range     :   0 - CH2 Off

1 - CH2 On

Default value   :   1 - CH2 On

tracking

Variable Type       short

Control name    :   Tracking

Description     :   Set the mode tracking On or

Off (AX2320 only). When On,

voltage and current output of

channel two is identical to

channel one and is controlled

by channel one .

Variable type   :   Integer

Valid range     :   0 - no tracking

1 - tracking

Default value   :   0 - no tracking

OCP

Variable Type       short

Control name    :   OCP

Description     :   The overcurrent programming

feature protects the load from

excessive output current when

enabled .

Variable type   :   Integer

Valid range     :   0 - OCP Off

1 - OCP On

Default value   :   0 - OCP Off

Return Value

Control name    : Error

Description     : This return value is placed in

the global variable ax2320_err.

Variable type   : Integer

Valid range     :

0     Success

-1     ID Instrument out of range

-2     CH1 out of range

-3     CH2 out of range

-4     Tracking out of range

-5     OCP out of range

230     Error writing to instrument

231     Error reading from instrument

232     Device not initialized

________________________________________________________________________________

ax2320_set_over_voltage

short ax2320_set_over_voltage (short IDInstrument, short channel,

double overvoltage);

Purpose

Panel name      :   Set Overvoltage

Description     :   Specify an overvoltage

protection to guard against

abnormal operating conditions.

Range : 0.000 to 35.000

When this function is tripped,

the unit disables the output

and the LCD displays "OV" .

For AX1320 , only CH1 is available.

Parameter List

IDInstrument

Variable Type       short

Control Name    : ID instrument

Description     : Accepts the instrument ID

returned by the Initialize

fonction to select the desired

instrument.

Valid Range     : 1 to Maximum Instance of this

instrument.

The Maximum Instances of this

instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system.The value is set

internally in the driver and is

typically 10.

Variable Type   : Integer

channel

Variable Type       short

Control name    :  Channel

Description     :  Indicates the channel to

which the overvoltage will

be sent.

Variable type   :  integer

Valid range     :  1   CH 1

2   CH 2 (AX2320)

Default value   :  1   CH 1

overvoltage

Variable Type       double

Control name    : Overvoltage

Description     : Set the overvoltage on CH1

or CH2 (AX2320). Values are in

volts (i.e. no millivolts).

When this function is tripped,

the unit disables the output

and the LCD displays "OV" .

Variable type   : Double

Valid range     : 0.000 to 35.000

Default value   : 35.000

Return Value

Control name    : Error

Description     : This return value is placed in

the global variable ax2320_err.

Variable type   : Integer

Valid range     :

0     Success

-1     ID Instrument out of range

-2     Channel out of range

-3     Overvoltage out of range

230     Error writing to instrument

231     Error reading from instrument

300     CH2 not available with AX1320 model.

________________________________________________________________________________

ax2320_set_voltage

short ax2320_set_voltage (short IDInstrument, short channel, double voltage);

Purpose

Panel name      :   Set Output Voltage

Description     :   Specify an output channel and

voltage in volts (i.e. no

millivolts): 0.000 to 32.000.

If the output voltage exceeds

the overvoltage of the power

supply, the unit will reduce

the output to zero and the LCD

will display "OV".

For AX1320 , only CH1 is available.

Parameter List

IDInstrument

Variable Type       short

Control Name    : ID instrument

Description     : Accepts the instrument ID

returned by the Initialize

fonction to select the desired

instrument.

Valid Range     : 1 to Maximum Instance of this

instrument.

The Maximum Instances of this

instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system.The value is set

internally in the driver and is

typically 10.

Variable Type   : Integer

channel

Variable Type       short

Control name    :  Channel

Description     :  Indicates the channel to

which the voltage will be sent.

Variable type   :  integer

Valid range     :  1   CH 1

2   CH 2 (AX2320)

Default value   :  1   CH 1

voltage

Variable Type       double

Control name    : Voltage

Description     : Set the voltage on CH1 or CH2

(AX2320).Values are in volts

(i.e. no millivolts).

If the output voltage exceeds

the overvoltage of the power

supply, the unit will reduce

the output to zero and the LCD

will display "OV".

Variable type   : Double

Valid range     : 0.000 to 32.000

Default value   : 5.000

Return Value

Control name    : Error

Description     : This return value is placed in

the global variable ax2320_err.

Variable type   : Integer

Valid range     :

0     Success

-1     ID Instrument out of range

-2     Channel out of range

-3     Voltage out of range

230     Error writing to instrument

231     Error reading from instrument

300     CH2 not available with AX1320 model.

________________________________________________________________________________

ax2320_write

short ax2320_write (short instrumentID, char writeBuffer[]);

Purpose

Panel Name:    Write To Instrument

Description:   Writes commands and queries to the

instrument to modify parameters

and query device settings.

Parameter List

instrumentID

Variable Type       short

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Default Value: 1

writeBuffer

Variable Type       char []

Control Name:  Write Buffer

Description:   The user can use this control to

send common commands and queries

to the instrument. This control

can also be used to write any

valid command to the instrument.

Variable Type: String (char *)

Default Value: "*RST"

NOTES: (1) The command or query to be sent to the

instrument may be a literal enclosed

in double quotes i.e. "*RST" or may be

contained in a variable of type

string.

(2) If the number of bytes to be written

is greater than the size of "cmd" in

the source code then the size of cmd

must be increased. The original size

of cmd is 50 bytes (cmd[50]) the

largest cmd may be declared as is

32,767 bytes.

Return Value

Control Name:  Error Code

Description:   Displays the results of the

function call.

Variable Type: Integer

Error Codes:

0   Success

-1   Instrument ID out of range

230   Error writing to instrument

232   Instrument not initialized