Metrix GX1304 Function Generator

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Metrix GX1304 Function Generator.

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For GPIB instrument drivers:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

For VXI instrument drivers:

-  the instrument is installed in the VXI mainframe and you are

using one of the following controller options:

Embedded controller

AT-MXI

MC-MXI

GPIB-VXI

-  the logical address supplied to the initialize function must

match the logical address of the instrument.

-  if you are using GPIB-VXI, there must be at least one unused

device name in the handler.

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error Codes:

Error codes are returned as the return value of each instrument driver

function. A program should examine this value after each call to an instrument

driver function to determine if any error occurred. Possible error codes and

their meanings are listed with the corresponding instrument driver function.

________________________________________________________________________________

How To Use This Document:

This document is intended to be used as a programming reference manual.

It describes each function in the

Metrix GX1304 Function Generator

instrument. The functions are listed in alphabetical order with a

description of the function, C syntax of the function, a description of each

parameter, and possible error codes.

________________________________________________________________________________

Function Tree Layout:

Metrix GX1304 Function Generator       Function Name:

Initialize                                  gx1304_init

Configure

Main generator                           gx1304_maing

Auxiliary generator                      gx1304_auxg

Amplitude Modulation

AM                                    gx1304_am

VCA                                   gx1304_vca

Sum                                   gx1304_sum

Frequency Modulation

FM                                    gx1304_fm

Sweep                                 gx1304_vcf

Memory Operations

Store & Recall Setup                     gx1304_storecall

Transfer Setup                           gx1304_transfer

Utility Functions

Write To Instrument                      gx1304_write

Read Instrument Data                     gx1304_read

Reset                                    gx1304_reset

Self-Test                                gx1304_self_test

Revision Query                           gx1304_revision_query

Close                                       gx1304_close

________________________________________________________________________________

Metrix GX1304 Function Generator

Instrument Name: GX1304 Function Generator

Description:     This instrument module provides

programming support for the

GX1304.

The module is divided into the

following functions:

Functions/Classes:

(1) Initialize:

Initialize the instrument and set to default

configuration.

(2) Configure : (Class)

Provides functions to configure the Main and

Auxiliary generators as well as the Amplitude

and Frequency Modulation (Sub Classes).

(3) Memory Operations : (Class)

Provides functions to Save and Recall the

instrument Setup (stored in the instrument)

or to Transfer instrument Setup (checksum

binary blocks).

(4) Utility Functions: (Class)

Provides lower level functions to communicate

with the instrument, and change instrument

parameters.

(5) Close:

Take the instrument offline.

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

gx1304_am

short gx1304_am (short IDInstrument, short function, double modfreqHz,

double AMDepth);

Purpose

Panel name : AM (Amplitude Modulation)

Description: Sets the following parameters:

* function

* modulation frequency

* depth

Parameter List

IDInstrument

Variable Type       short

Control Name    : ID instrument

Description     : Accepts the instrument ID

returned by the Initialize

fonction to select the desired

instrument.

Valid Range     : 1 to Maximum Instance of this

instrument.

The Maximum Instances of this

instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system.The value is set

internally in the driver and is

typically 10.

Variable Type   : Integer

function

Variable Type       short

Control label  : Function

Description    : Specifies the shape of the

modulating signal (auxiliary

generator).

Variable type  : integer

Variable range : 0 to 4

0 : AM off

1 : Sine

2 : Triangle

3 : Square

4 : Ext Signal

modfreqHz

Variable Type       double

Control label  : Mod Freq (Hz)

Description    : Specifies the Modulation

frequency (Hz) .

Variable type  : real

Variable range : 0.005 to 50 000.0

AMDepth

Variable Type       double

Control label  : Depth

Description    : Specifies the Amplitude Modu-

lation frequency .

Variable type  : real

Variable range : 0.0 to 99.0

Return Value

Control Name:  Error Code

Description:   Displays the results of the

function call.

Variable Type: Integer

Error Codes:

0   Success

-1   ID instrument out of range

-2   Function out of range

-3   Frequency out of range

-4   Depth out of range

230   Error writing to instrument

232   Instrument not initialized

________________________________________________________________________________

gx1304_auxg

short gx1304_auxg (short IDInstrument, short function, double frequencyHz,

double amplitudeVpp, double symetry);

Purpose

Panel name : Auxiliary generator

Description: Sets the following parameters:

* function

* frequency

* amplitude

* symmetry

Parameter List

IDInstrument

Variable Type       short

Control Name    : ID instrument

Description     : Accepts the instrument ID

returned by the Initialize

fonction to select the desired

instrument.

Valid Range     : 1 to Maximum Instance of this

instrument.

The Maximum Instances of this

instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system.The value is set

internally in the driver and is

typically 10.

Variable Type   : Integer

function

Variable Type       short

Control label  : Function

Description    : Specifies the output signal .

Variable type  : integer

Variable range : 0 to 6

0 : Auxiliary Out off

1 : Sine

2 : Triangle

3 : Square

4 : Ramp

5 : Main Generator Frequency

Controlled Voltage (FCV)

6 : Main Generator Amplitude

Controlled Voltage (ACV)

frequencyHz

Variable Type       double

Control label  : Frequency

Description    : Specifies the frequency (Hz)

Variable type  : real

Variable range : 0.005 to 50 000.0

amplitudeVpp

Variable Type       double

Control label  : Amplitude

Description    : Specifies the AC amplitude

in terms of peak to peak

voltage (V)

Variable type  : real

Variable range :

0.002 to 20.0 emf for sine,triangle and square

symetry

Variable Type       double

Control label  : Symmetry

Description    : Specifies the symmetry of

the auxiliary generator (%) .

Variable type  : real

Variable range : 5.0 to 95.0

Return Value

Control Name:  Error Code

Description:   Displays the results of the

function call.

Variable Type: Integer

Error Codes:

0   Success

-1   ID instrument out of range

-2   Function out of range

-3   Frequency out of range

-4   Amplitude out of range

-5   Symmetry out of range

230   Error writing to instrument

232   Instrument not initialized

________________________________________________________________________________

gx1304_close

short gx1304_close (short IDInstrument);

Purpose

Function name : Close

Description   : Close the instrument module

and place the instrument in

local mode .

Parameter List

IDInstrument

Variable Type       short

Control Name    : ID instrument

Description     : Accepts the instrument ID

returned by the Initialize

fonction to select the desired

instrument.

Valid Range     : 1 to Maximum Instance of this

instrument.

The Maximum Instances of this

instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system.The value is set

internally in the driver and is

typically 10.

Variable Type   : Integer

Return Value

Control Name:  Error Code

Description:   Displays status relating to the

function call.

Variable Type: Integer

Error Codes:

0   Success

-1   Instrument ID out of range

221   Unable to close instrument

232   Instrument not initialized

234   Error in placing instrument in local

mode.

________________________________________________________________________________

gx1304_fm

short gx1304_fm (short IDInstrument, short function, double modfreqHz,

double FMdeviationHz);

Purpose

Panel name : FM

(Frequency Modulation)

Description: Sets the following parameters:

* function

* modulation frequency

* deviation

Parameter List

IDInstrument

Variable Type       short

Control Name    : ID instrument

Description     : Accepts the instrument ID

returned by the Initialize

fonction to select the desired

instrument.

Valid Range     : 1 to Maximum Instance of this

instrument.

The Maximum Instances of this

instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system.The value is set

internally in the driver and is

typically 10.

Variable Type   : Integer

function

Variable Type       short

Control label  : Function

Description    : Specifies the shape of the

modulating signal (auxiliary

generator).

Variable type  : integer

Variable range : 0 to 4

0 : FM off

1 : Sine

2 : Triangle

3 : Square

4 : Ext Signal

modfreqHz

Variable Type       double

Control label  : Mod Freq (Hz)

Description    : Specifies the Modulation

frequency (Hz) .

Variable type  : real

Variable range : 0.005 to 50 000.0

FMdeviationHz

Variable Type       double

Control label  : FM Deviation (Hz)

Description    : Specifies the frequency

deviation .

Variable type  : real

Variable range : 0.001 to 13 000 000.0

Return Value

Control Name:  Error Code

Description:   Displays the results of the

function call.

Variable Type: Integer

Error Codes:

0   Success

-1   ID instrument out of range

-2   Function out of range

-3   Mode frequency out of range

-4   FM deviation out of range

230   Error writing to instrument

232   Instrument not initialized

301   FM deviation >= MAIN frequency

302   FM deviation+main frequency >13 Mhz

________________________________________________________________________________

gx1304_init

short gx1304_init (short GPIBAddress, short IDQuery, short resetDevice,

short *IDInstrument);

Purpose

Panel Name:    Initialize

Description:   Initializes the instrument in the

following ways:

- opens the instrument and sets

its GPIB address to the speci-

fied value

- performs an identification query

on the instrument

- sets the instrument to a default

configuration (See Note below)

- returns an instrument ID which

is used to identify the

instrument in subsequent calls.

Note : Default Configuration

Main gen : 10Khz 50% Sine 1Vpp Output off

Aux gen  :  1Khz 50% Sine 1Vpp Output off

All trigger and modulation menus cleared

Parameter List

GPIBAddress

Variable Type       short

Control Name:  GPIB Address

Description:   Specifies the address on the GPIB

to be used to communicate

with the instrument.  This value

must correspond to the current

internal setting of the

instrument.

Variable Type: Integer

Valid Range:   0 to 30

Default Value: 1

NOTE:  The Default Value of this control should

be set to the factory default of the

instrument, however if your instrument's

actual setting is different this control's

default should be changed.

IDQuery

Variable Type       short

Control Name:  ID Query

Description:   Specifies if an ID Query is sent

to the instrument during the

initialization procedure.

Variable Type: Integer

Valid Range:   0 - Skip Query

1 - Do Query (Default)

NOTE:  Under normal circumstances the ID Query

ensures that the instrument initialized

over the GPIB is the type supported by

this driver.  However circumstances may

arise where it is undesirable to send an

ID Query to the instrument.  In those

cases; set this control to "Skip Query"

and this function will initialize the

GPIB and the Command arrays in the driver,

without doing an ID Query.

resetDevice

Variable Type       short

Control Name:  Reset Device

Description:   Specifies if the instrument is to

be reset to its power-on settings

during the initialization

procedure.

Variable Type: Integer

Valid Range:   0 - Don't Reset

1 - Reset Device (Default)

NOTE: If you do not want the instrument reset.

Set this control to "Don't Reset" while

initializing the instrument.

IDInstrument

Variable Type       short (passed by reference)

Control name    : ID Instrument

Description     : Returns an instrument ID that

is used in all subsequent

function calls to select the

instrument at the Address

specified.

If more than one instrument of

the same type is used,this value

will be used to differentiate

beetween them. The first

instrument initialized will be

assigned an Instrument ID of 1.

Variable Type   : Integer

Return Value

Control Name:  Error Code

Description:   Displays the results of the

function call.

Variable Type: Integer

Error Codes:

0   Success

-1   GPIB Address out of range

-2   ID Query out of range

-3   Reset Device out of range

220   Unable to open instrument

223   Instrument identification query failed

230   Error writing to instrument

231   Error reading from instrument

233   Error configuring GPIB address

240   Instrument timed out

________________________________________________________________________________

gx1304_maing

short gx1304_maing (short IDInstrument, short function, double frequencyHz,

double amplitudeVpp, double symmetry, double DCOffsetV,

double startStopPhase);

Purpose

Panel name : Main generator

Description: Sets the following parameters:

* function

* frequency

* amplitude

* symmetry

* offset

* phase

Parameter List

IDInstrument

Variable Type       short

Control Name    : ID instrument

Description     : Accepts the instrument ID

returned by the Initialize

fonction to select the desired

instrument.

Valid Range     : 1 to Maximum Instance of this

instrument.

The Maximum Instances of this

instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system.The value is set

internally in the driver and is

typically 10.

Variable Type   : Integer

function

Variable Type       short

Control label  : Function

Description    : Specifies the output signal .

Variable type  : integer

Variable range : 0 to 8

0 : Main Out off

1 : Sine

2 : Triangle

3 : Square

4 : Offset

5 : Peak positive voltage

6 : Peak negative voltage

7 : Positive going pulse

8 : Negative going pulse

frequencyHz

Variable Type       double

Control label  : Frequency

Description    : Specifies the frequency (Hz)

Variable type  : real

Variable range : 0.01 to 13 000 000.0

amplitudeVpp

Variable Type       double

Control label  : Amplitude

Description    : Specifies the AC amplitude

in terms of peak to peak

voltage (V)

Variable type  : real

Variable range :

0.002 to 20.0 emf for sine,triangle and square

0.001 to 10.0 emf for unipolar waveform

symmetry

Variable Type       double

Control label  : Symmetry

Description    : Specifies the symmetry of

the main generator (%) .

Variable type  : real

Variable range : 5.0 to 95.0

DCOffsetV

Variable Type       double

Control label  : Offset

Description    : Specifies the DC offset of

the main generator (emf V) .

Variable type  : real

Variable range : -10.0 to +10.0

startStopPhase

Variable Type       double

Control label  : Phase

Description    : Specifies the start phase of

the main generator (�) .

Variable type  : real

Variable range : - 360.0 to +360.0

Return Value

Control Name:  Error Code

Description:   Displays the results of the

function call.

Variable Type: Integer

Error Codes:

0   Success

-1   ID instrument out of range

-2   Function out of range

-3   Frequency out of range

-4   Amplitude out of range

-5   Symmetry out of range

-6   DCO out off range

-7   Phase out of range

230   Error writing to instrument

232   Instrument not initialized

301    Peak amplitude out of range

302    FM deviation >= main frequency

303    FM deviation+main frequency >13 Mhz

________________________________________________________________________________

gx1304_read

short gx1304_read (short instrumentID, short numberBytesToRead,

char readBuffer[], short *numBytesRead);

Purpose

Panel Name:    Read Instrument Data

Description:   Reads data from instrument buffer

and returns it to the specified

variable in memory.

NOTES: (1) Because the instrument may return both

numeric and text data in response to

queries this function returns the data

in string format.

(2) If valid data is NOT available at the

instrument's output buffer when this

function is called the instrument will

hang up and the function will not

return until it times out.  If the

time-out is disabled this function

will hang indefinitely and it may be

neccessary to reboot the computer to

break out.

Parameter List

instrumentID

Variable Type       short

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Default Value: 1

numberBytesToRead

Variable Type       short

Control Name:  Number of Bytes to Read

Description:   The number of bytes specified by

this control should be greater

than or equal to the number of

bytes which are to be read from

the instrument. If the actual

number of bytes to be read is

greater than the number this

control specifies then multiple

reads will be necessary to empty

the instrument's output buffer.

If the instrument's output buffer

is not emptied the instrument may

return invalid data when future

reads are performed.

Variable Type: Integer

Default Value: 50 (See NOTE 2)

NOTES: (1) If NO DATA is available at the

instrument's output buffer when this

function is called the instrument will

hang up and the function will not

return until it times out.  If the

time-out is disabled, this function

will hang indefinitely and it may be

necessary to reboot the computer.

(2) If the number of bytes expected is

greater than 50 the value of this

control may be increased. If the

actual number of bytes read is larger

than the declared size of the read

buffer a run-time error will be

generated.

readBuffer

Variable Type       char []

Control Name:  Read Buffer

Description:   The incoming data from the

instrument is placed into this

variable.

Variable Type: String (char *)

NOTES: (1) Because the Instrument may return both

numeric and text data in response to

queries this function returns the data

in string form.

(2) This function does not overwrite any

old data left in the string variable

from the last time the function was

called. String data in LabWindows is

terminated with an ASCII null(Hex 0)

and string manipulation functions will

only recognize data before the ASCII

null.

(3) The declared size of the string

variable must be greater than the

actual number of bytes read from the

instrument if it is not a run-time

error will be generated.

numBytesRead

Variable Type       short (passed by reference)

Control Name:  Number of Bytes Read

Description:   This variable contains the actual

number of bytes read from the

instrument. This is the value

which is returned by ibrd in the

global variable ibcnt.

Variable Type: Integer *

NOTE: If the actual number of bytes read is less

than the number of bytes specified in

number of bytes to read then the output

buffer is probably empty. If the number of

bytes read is 0 and the instrument returns

a 231 error code the read function failed.

The most probable cause for the read to

fail is because there was no data available

at the instrument's output buffer.

Return Value

Control Name:  Error Code

Description:   Displays the results of the

function call.

Variable Type: Integer

Error Codes:

0   Success

-1   Instrument ID out of range

231   Error reading from instrument

232   Instrument not initialized

________________________________________________________________________________

gx1304_reset

short gx1304_reset (short instrumentID);

Purpose

Panel Name:    Reset

Description:   This function resets the

instrument to its default state.

Parameter List

instrumentID

Variable Type       short

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Default Value: 1

Return Value

Control Name:  Error Code

Description:   Displays status relating to the

function call.

Variable Type: Integer

Error Codes:

0   Success

-1   Instrument ID out of range

230   Error writing to instrument

232   Instrument not initialized

________________________________________________________________________________

gx1304_revision_query

short gx1304_revision_query (short instrumentID, char driverRevision[],

char firmwareRevision[]);

Purpose

Panel Name:    Revision Query

Description:   This function returns the revision

numbers of the instrument driver

and instrument firmware.

Parameter List

instrumentID

Variable Type       short

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Default Value: 1

driverRevision

Variable Type       char []

Control Name:  Driver Revision

Description:   This control returns the

Instrument Driver Revision.

Variable Type: String (char *)

firmwareRevision

Variable Type       char []

Control Name:  Instrument Firmware Revision

Description:   This control returns the

Instrument's Firmware Revision.

Variable Type: String (char *)

Return Value

Control Name:  Error Code

Description:   Displays status relating to the

function call.

Variable Type: Integer

Error Codes:

0   Success

-1   Instrument ID out of range

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

________________________________________________________________________________

gx1304_self_test

short gx1304_self_test (short instrumentID, short *passFailCode,

char selfTestMessage[]);

Purpose

Panel Name:    Self Test

Description:   This function runs the

instrument's self test routine and

returns the test result(s).

Parameter List

instrumentID

Variable Type       short

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Default Value: 1

passFailCode

Variable Type       short (passed by reference)

Control Name:  Pass/Fail Code

Description:   This is the value returned by the

self test.  Zero means success.

For any other code, see the

Device's operator's manual.

Variable Type: Integer *

selfTestMessage

Variable Type       char []

Control Name:  Self Test Message

Description:   This is the string returned by the

self test. See the device's

operation manual for an

explanation of the string's

contents.

Variable Type: String (char *)

Return Value

Control Name:  Error Code

Description:   Displays status relating to the

function call.

Variable Type: Integer

Error Codes:

0   Success

-1   Instrument ID out of range

226   Error polling instrument

230   Error writing to instrument

231   Error reading from instrument

232   Instrument not initialized

236   Error interpreting instrument response

239   Error configuring time out

240   Instrument timed out

________________________________________________________________________________

gx1304_storecall

short gx1304_storecall (short IDInstrument, short function, short storeNumber);

Purpose

Panel name : Store & Recall Setup

Description: Saves or recalls the specified

(1 to 51) instrument setup

(stored in the instrument).

Parameter List

IDInstrument

Variable Type       short

Control Name    : ID instrument

Description     : Accepts the instrument ID

returned by the Initialize

fonction to select the desired

instrument.

Valid Range     : 1 to Maximum Instance of this

instrument.

The Maximum Instances of this

instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system.The value is set

internally in the driver and is

typically 10.

Variable Type   : Integer

function

Variable Type       short

Control label  : Function

Description    : Saves or recalls the complete

instrument setup .

Variable type  : integer

Variable range : 0 to 1

0 : Save

1 : Recall

storeNumber

Variable Type       short

Control label  : Store Number

Description    : Specifies the store or recall

number.

Variable type  : integer

Variable range : 1 to 51

Return Value

Control Name:  Error Code

Description:   Displays the results of the

function call.

Variable Type: Integer

Error Codes:

0   Success

-1   ID instrument out of range

-2   Function out of range

-3   Store number out of range

230   Error writing to instrument

232   Instrument not initialized

________________________________________________________________________________

gx1304_sum

short gx1304_sum (short IDInstrument, short function, double modfreqHz,

double SUMamplVpp);

Purpose

Panel name : Sum

Description: Sets the following parameters:

* function

* frequency

* amplitude

Parameter List

IDInstrument

Variable Type       short

Control Name    : ID instrument

Description     : Accepts the instrument ID

returned by the Initialize

fonction to select the desired

instrument.

Valid Range     : 1 to Maximum Instance of this

instrument.

The Maximum Instances of this

instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system.The value is set

internally in the driver and is

typically 10.

Variable Type   : Integer

function

Variable Type       short

Control label  : Function

Description    : Specifies the signal added to

the main generator .

Variable type  : integer

Variable range : 0 to 4

0 : SUM off

1 : Sine

2 : Triangle

3 : Square

4 : External

modfreqHz

Variable Type       double

Control label  : Mod freq (Hz)

Description    : Specifies the frequency of

the signal added to the main

generator .

Variable type  : real

Variable range : 0.005 to 50 000.0

SUMamplVpp

Variable Type       double

Control label  : Sum Ampl (Vpp)

Description    : Specifies the amplitude of

the signal added to the main

generator .

Variable type  : real

Variable range : 0.002 to 20.0

Return Value

Control Name:  Error Code

Description:   Displays the results of the

function call.

Variable Type: Integer

Error Codes:

0   Success

-1   ID instrument out of range

-2   Function out of range

-3   Frequency out of range

-4   Amplitude out of range

230   Error writing to instrument

232   Instrument not initialized

301   Peak Amplitude out of range

(Sum Vpp/2 + Main Vpp/2 +DCO < 10.0)

________________________________________________________________________________

gx1304_transfer

short gx1304_transfer (short IDInstrument, short function, short storeNumber,

char filename[]);

Purpose

Panel name : Transfer Setup

Description: Allows complete instrument

setup to be transferred rapidly

to and from the controller in

the form of checksummed binary

blocks .

The data is read from the

GX1304 by sending the STORE?

query with the required number

as the parameter . The response

to this query is the correspon-

ding STORE command with the

store number parameter and the

binary block with its length

declaration header .

i.e. the response to STORE? 2

is STORE2,#3386<binary data>LF-

with END-

The response is stored in the

specified file and returned to

the instrument as later requi-

red . Note that the binary data

contains a checksum and should

not be modified , but the store

number may be altered as requi-

red .

With the large number of stores available

in th GX1304 , many test steps may be

downloaded from a configuration file by

the controller in advance , with the

actual test sequence consisting simply of

Writing commands . This minimizes bus

traffic and maximises responses speed

during the test .

Parameter List

IDInstrument

Variable Type       short

Control Name    : ID instrument

Description     : Accepts the instrument ID

returned by the Initialize

fonction to select the desired

instrument.

Valid Range     : 1 to Maximum Instance of this

instrument.

The Maximum Instances of this

instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system.The value is set

internally in the driver and is

typically 10.

Variable Type   : Integer

function

Variable Type       short

Control label  : Function

Description    : Allows a complete instrument

setup to be transferred rapi-

dly to or from the controller

in the form of checksummed

binary blocks .

Variable type  : integer

Variable range : 0 to 1

0 : Read to file

1 : Write from file

storeNumber

Variable Type       short

Control label  : Store number

Description    : In reading mode it specifies

the store number of the setup

instrument to be red .

In writing mode , the store

number is not used (See Panel

Help for more explanations).

Variable type  : integer

Variable range : 1 to 50

filename

Variable Type       char []

Control label  : Filename

Description    : Specifies the filename to be

red (Write from file) or

written (Read to file) .

Variable type  : String

Return Value

Control Name:  Error Code

Description:   Displays the results of the

function call.

Variable Type: Integer

Error Codes:

0   Success

-1   ID instrument out of range

-2   Function out of range

-3   Store number out of range

228   Error writing to instrument from file

229   Error reading from instrument to file

230   GPIB write error

232   Instrument not initialized

301   Error on opening file

302   Bad file size ( <> 387 or 388)

________________________________________________________________________________

gx1304_vca

short gx1304_vca (short IDInstrument, short function, double startAmplV,

double stopAmplV, double sweepTimeS);

Purpose

Panel name : VCA

(Voltage Controlled Amplitude)

Description: Sets the following parameters:

* function

* start amplitude

* stop amplitude

* sweep time

Parameter List

IDInstrument

Variable Type       short

Control Name    : ID instrument

Description     : Accepts the instrument ID

returned by the Initialize

fonction to select the desired

instrument.

Valid Range     : 1 to Maximum Instance of this

instrument.

The Maximum Instances of this

instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system.The value is set

internally in the driver and is

typically 10.

Variable Type   : Integer

function

Variable Type       short

Control label  : Function

Description    : Specifies the signal (inter-

nal or external) and the law

which controls the main gene-

rator amplitude .

Variable type  : integer

Variable range : 0 to 4

0 : VCA off

1 : Int Lin

2 : Int Log

3 : Ext Lin

4 : Ext Log

startAmplV

Variable Type       double

Control label  : Start Amplitude

Description    : Specifies the Start amplitude

of the main output in terms

of peak to peak voltage (V).

Variable type  : real

Variable range : 0.002 to 20.0

stopAmplV

Variable Type       double

Control label  : Stop Amplitude

Description    : Specifies the Stop amplitude

of the main output in terms

of peak to peak voltage (V).

Variable type  : real

Variable range : 0.002 to 20.0

sweepTimeS

Variable Type       double

Control label  : Sweep Time (S)

Description    : Specifies the sweep time

of the VCA mode .

Variable type  : real

Variable range : 20E-06 to 200.0

Return Value

Control Name:  Error Code

Description:   Displays the results of the

function call.

Variable Type: Integer

Error Codes:

0   Success

-1   ID instrument out of range

-2   Function out of range

-3   Start voltage out of range

-4   Stop voltage out of range

-5   Sweep time out of range

230   Error writing to instrument

232   Instrument not initialized

301   VCA stop/VCA start ratio out of range

________________________________________________________________________________

gx1304_vcf

short gx1304_vcf (short IDInstrument, short function, double startFreqHz,

double stopFreqHz, double sweepTimeS);

Purpose

Panel name : VCF

(Voltage Controlled Frequency)

Description: Sets the following parameters:

* function

* start frequency

* stop frequency

* sweep time

Parameter List

IDInstrument

Variable Type       short

Control Name    : ID instrument

Description     : Accepts the instrument ID

returned by the Initialize

fonction to select the desired

instrument.

Valid Range     : 1 to Maximum Instance of this

instrument.

The Maximum Instances of this

instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system.The value is set

internally in the driver and is

typically 10.

Variable Type   : Integer

function

Variable Type       short

Control label  : Function

Description    : Specifies the signal (inter-

nal or external) and the law

which controls the main gene-

rator frequency .

Variable type  : integer

Variable range : 0 to 4

0 : VCF off

1 : Int Lin

2 : Int Log

3 : Ext Lin

4 : Ext Log

startFreqHz

Variable Type       double

Control label  : Start Frequency (Hz)

Description    : Specifies the Start frequency

of the main output .

Variable type  : real

Variable range : 0.001 to 13000000.0

stopFreqHz

Variable Type       double

Control label  : Stop Frequency (Hz)

Description    : Specifies the Stop frequency

of the main output .

Variable type  : real

Variable range : 0.001 to 13000000.0

sweepTimeS

Variable Type       double

Control label  : Sweep Time (S)

Description    : Specifies the sweep time

of the VCF mode .

Variable type  : real

Variable range : 20E-06 to 200.0

Return Value

Control Name:  Error Code

Description:   Displays the results of the

function call.

Variable Type: Integer

Error Codes:

0   Success

-1   ID instrument out of range

-2   Function out of range

-3   Start frequency out of range

-4   Stop frequency out of range

-5   Sweep time out of range

230   Error writing to instrument

232   Instrument not initialized

301   VCF stop/VCF start ratio out of range

________________________________________________________________________________

gx1304_write

short gx1304_write (short instrumentID, char writeBuffer[]);

Purpose

Panel Name:    Write To Instrument

Description:   Writes commands and queries to the

instrument to modify parameters

and query device settings.

Parameter List

instrumentID

Variable Type       short

Control Name:  Instrument ID

Description:   Accepts the Instrument ID returned

by the Initialize function to

select the desired instrument.

Variable Type: Integer

Valid Range:   1 to  Maximum Instances of

this Instrument

The Maximum Instances of this

Instrument is the maximum number

of instruments that can be

controlled by this driver in a

single system. The value is set

internally in the driver. The

typical value is 10.

Default Value: 1

writeBuffer

Variable Type       char []

Control Name:  Write Buffer

Description:   The user can use this control to

send common commands and queries

to the instrument. This control

can also be used to write any

valid command to the instrument.

Variable Type: String (char *)

Default Value: "*RST"

NOTES: (1) The command or query to be sent to the

instrument may be a literal enclosed

in double quotes i.e. "*RST" or may be

contained in a variable of type

string.

(2) If the number of bytes to be written

is greater than the size of "cmd" in

the source code then the size of cmd

must be increased. The original size

of cmd is 50 bytes (cmd[50]) the

largest cmd may be declared as is

32,767 bytes.

Return Value

Control Name:  Error Code

Description:   Displays the results of the

function call.

Variable Type: Integer

Error Codes:

0   Success

-1   Instrument ID out of range

230   Error writing to instrument

232   Instrument not initialized