Metrix MTX1054B DSO 150MS/s (VISA)

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Metrix MTX1054B DSO 150MS/s (VISA).

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For GPIB instrument drivers:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

For VXI instrument drivers:

-  the instrument is installed in the VXI mainframe and you are

using one of the following controller options:

Embedded controller

MXI

MXI2

GPIB-VXI

-  the logical address supplied to the initialize function must

match the logical address of the instrument.

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error and Status Information:

Each function in this instrument driver returns a status code that

either indicates success or describes an error or warning condition.

Your program should examine the status code from each call to an

instrument driver function to determine if an error occurred.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

The description of each instrument driver function lists possible error

codes and their meanings

________________________________________________________________________________

How To Use This Document:

Use this document as a programming reference manual.

It describes each function in the

Metrix MTX1054B DSO 150MS/s (VISA)

instrument.  The functions appear in alphabetical order, with a description

of the function and its C syntax, a description of each parameter, and a list

of possible error codes.

________________________________________________________________________________

Function Tree Layout:

Class/Panel Name:                          Function Name:

Initialize                              mtx1054B_init

Application Functions

Example                              mtx1054B_example

Configure Functions

Vertical Setup                       mtx1054B_verticalSetup

Horizontal

Horizontal Setup                  mtx1054B_horizontalSetup

Horizontal Display                mtx1054B_horizontalDisplay

FFT                               mtx1054B_FFT

Trigger

General Trigger                   mtx1054B_setGeneralTrigger

Pulse Trigger                     mtx1054B_setPulsTrigger

Delayed Trigger                   mtx1054B_setDelayedTrigger

Count Trigger                     mtx1054B_setCountTrigger

Video Trigger                     mtx1054B_setVideoTrigger

Display

Options                           mtx1054B_setOptionDisplay

Waveform                          mtx1054B_setWaveformDisplay

SPO Mode                          mtx1054B_setSPOOptions

Cursors                           mtx1054B_setCursors

Action/Status Functions

Autoset                              mtx1054B_runAutoset

Run/Stop                             mtx1054B_runAcquisition

Data

Read Cursors                         mtx1054B_readCursors

Read Measurements                    mtx1054B_readMeasurement

Waveform

Read Channel To Array             mtx1054B_readChToArr

Read Channel To File              mtx1054B_readChToFile

Read File To Array                mtx1054B_readFileToArr

Transfer

Save/Recall Setup To/From File    mtx1054B_saveSetup

Utility Functions

Write To Instrument                  mtx1054B_writeInstrData

Read Instrument Data                 mtx1054B_readInstrData

Reset                                mtx1054B_reset

Self-Test                            mtx1054B_selfTest

Error-Query                          mtx1054B_errorQuery

Error Message                        mtx1054B_errorMessage

Revision Query                       mtx1054B_revisionQuery

Close                                   mtx1054B_close

________________________________________________________________________________

Metrix MTX1054B DSO 150MS/s (VISA)

This instrument module provides programming support for the Metrix

MTX1054 (VISA).  The module is divided into the following functions:

Functions/Classes:

(1) Initialize:

This function initializes the instrument and sets it to a default

configuration.

(2) Application Functions: (Class)

This class contains high-level, test and measurement routines.  These

examples call other instrument driver functions to configure, start, and

read from the instrument.

(3) Configuration Functions: (Class)

This class of functions configures the instrument by setting acquisition

and system configuration parameters.

(4) Action/Status Functions: (Class)

This class of functions begins or terminates an acquisition. It also

provides functions which allow the user to determine the current status

of the instrument.

(5) Data Functions: (Class)

This class of functions transfers data to or from the instrument.

(6) Utility Functions: (Class)

This class of functions provides lower level functions to communicate

with the instrument, and change instrument parameters.

(7) Close:

This function takes the instrument offline.

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

mtx1054B_close

ViStatus mtx1054B_close (ViSession instrumentHandle);

Purpose

This function performs the following operations:

viClose (instrSession) and viClose (rmSession).

Notes:

(1) The instrument must be reinitialized to use it again.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_errorMessage

ViStatus mtx1054B_errorMessage (ViSession instrumentHandle,

ViStatus errorCode,

ViChar _VI_FAR errorMessage[]);

Purpose

This function takes the Status Code returned by the instrument driver

functions, interprets it and returns it as a user readable string.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  VI_NULL

errorCode

Variable Type       ViStatus

This control accepts the Status Code returned from the instrument

driver functions.

Default Value:

0 - VI_SUCCESS

errorMessage

Variable Type       ViChar[]

This control returns the interpreted Status Code as a user readable

message string.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

3FFF0085  Unknown status.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_errorQuery

ViStatus mtx1054B_errorQuery (ViSession instrumentHandle,

ViPInt32 errorCode,

ViChar _VI_FAR errorMessage[]);

Purpose

This function reads an error code from the instrument's error queue.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

errorCode

Variable Type       ViInt32 (passed by reference)

This control returns the error code read from the instrument's error

queue.

errorMessage

Variable Type       ViChar[]

This control returns the error message string read from the

instrument's error message queue.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_example

ViStatus mtx1054B_example (ViSession instrumentHandle, ViInt16 source,

ViInt16 time_base, ViInt16 voltageRange,

ViInt16 coupling,

ViReal64 _VI_FAR waveformArray[],

ViPInt32 number_of_samples, ViPReal64 xStart,

ViPReal64 xIncrement);

Purpose

This function is an example of this driver's functions use.

The instrument is set to the single shot mode.

User can change the source channel, the timebase, the input voltage range

and the input coupling.

A trigger is then sent to the instrument.

The function waits until the acquisition is completed.

Channel is read out from the instrument.

Waveform is interpolated if needed.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

source

Variable Type       ViInt16

This control selects the source channel.

Valid Values:

1 - CH1  (Default Value)

2 - CH2

3 - CH3

4 - CH4

time_base

Variable Type       ViInt16

This control selects the main timebase (A) value.

Valid Values:

0  - 1ns

1  - 2ns

2  - 5ns

3  - 10ns

4  - 20ns

5  - 50ns

6  - 100ns

7  - 200ns

8  - 500ns

9  - 1s

10 - 2s

11 - 5s

12 - 10s

13 - 20s

14 - 50s

15 - 100s

16 - 200s

17 - 500s

18 - 1ms

19 - 2ms

20 - 5ms

21 - 10ms

22 - 20ms

23 - 50ms

24 - 100ms

25 - 200ms

26 - 500ms

27 - 1s

28 - 2s

29 - 5s

30 - 10s

31 - 20s

32 - 50s

voltageRange

Variable Type       ViInt16

This control sets vertical sense for the specified channel.

Valid Values:

0  - 2mV/div

1  - 5mV/div

2  - 10mV/div

3  - 20mV/div

4  - 50mV/div

5  - 100mV/div

6  - 200mV/div

7  - 500mV/div

8  - 1V/div

9  - 2V/div

10 - 5V/div

11 - 10V/div  (Default Value)

12 - 20V/div

13 - 50V/div

14 - 100V/div

15 - 200V/div

16 - 500V/div

17 - 1kV/div

Note : This control value must be consistent with the probe value.

See the Digiscope 2000 Operating Instructions manual for more

details. If it is inconsistent an error is generated :

VI_ERROR_INCONSISTENT_PARAMETER7 (0xBFFC0907)

coupling

Variable Type       ViInt16

This control selects the input coupling of the selected channel.

Valid Values:

0 - AC  (Default Value)

1 - DC

2 - GND

waveformArray

Variable Type       ViReal64[]

This control specifies the name of the array in which to place the

waveform data.

Notes:

(1) The Real Array must be large enough to hold the requested points.

number_of_samples

Variable Type       ViInt32 (passed by reference)

This control returns the number of valid points read from the

selected channel.

xStart

Variable Type       ViReal64 (passed by reference)

This control returns the time of the first data point in seconds

(with respect to the trigger point).

xIncrement

Variable Type       ViReal64 (passed by reference)

This control returns the incrementing value in seconds for the X

axis.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Channel) out of range.

BFFC0003  Parameter 3 (Timebase) out of range.

BFFC0004  Parameter 4 (Voltage Range) out of range.

BFFC0005  Parameter 5 (Coupling) out of range.

BFFC0803  Error scanning response.

BFFC0900  Instrument specific error.

BFFC090C  Channel is not active.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_FFT

ViStatus mtx1054B_FFT (ViSession instrumentHandle, ViBoolean FFT,

ViInt16 calculate, ViBoolean scale);

Purpose

This function configures the FFT mode.

The following settings are available :

- FFT ON or OFF

- Calculate

- Scale

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

FFT

Variable Type       ViBoolean

This control valid the FFT mode.

Valid Values:

VI_OFF (0) - OFF  (Default Value)

VI_ON  (1) - ON

calculate

Variable Type       ViInt16

This control selects the calculate mode.

Valid Values:

0 - Rectangular  (Default Value)

1 - Hamming

2 - Hanning

3 - Blackman

scale

Variable Type       ViBoolean

This control selects the Scale.

Valid Values:

VI_OFF (0) - Log scale   (Default Value)

VI_ON  (1) - inear scale

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Trigger Source) out of range.

BFFC0003  Parameter 3 (Standard) out of range.

BFFC0004  Parameter 4 (Polarity) out of range.

BFFC0005  Parameter 5 (Field) out of range.

BFFC0006  Parameter 6 (line) out of range.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFC0906  Parameter 6 (Line) is inconsistent.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_horizontalDisplay

ViStatus mtx1054B_horizontalDisplay (ViSession instrumentHandle,

ViInt16 trace, ViInt16 x,

ViInt16 y);

Purpose

This function configures the horizontal display of the instrument.

The following settings are available :

- Trace

- X (for XY mode)

- Y (for XY mode)

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

trace

Variable Type       ViInt16

This control selects the traces to be displayed.

Valid Values:

0 - Oscilloscope  (Default Value)

1 - XY

x

Variable Type       ViInt16

This control selects the X input for XY mode.

Valid Values:

1 - CH1  (Default Value)

2 - CH2

3 - CH3

4 - CH4

y

Variable Type       ViInt16

This control selects the Y input for XY mode.

Valid Values:

1 - CH1  (Default Value)

2 - CH2

3 - CH3

4 - CH4

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Trace) out of range.

BFFC0003  Parameter 3 (X) out of range.

BFFC0004  Parameter 4 (Y) out of range.

BFFC0005  Parameter 5 (Roll) out of range.

BFFC0006  Parameter 6 (Cumul) out of range.

BFFC0803  Error scanning response.

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_horizontalSetup

ViStatus mtx1054B_horizontalSetup (ViSession instrumentHandle,

ViInt16 range,

ViReal64 run_after_delay);

Purpose

This function configures the horizontal settings of the instrument.

The following settings are available :

- Pretrigger

- Range (the main timebase)

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

range

Variable Type       ViInt16

This control selects the main timebase  value.

Valid Values:

0  - 1ns

1  - 2ns

2  - 5ns

3  - 10ns

4  - 20ns

5  - 50ns

6  - 100ns

7  - 200ns

8  - 500ns - (Default Value)

9  - 1s

10  - 2s

11 - 5s

12 - 10s

13 - 20s

14 - 50s

15 - 100s

16 - 200s

17 - 500s

18 - 1ms

19 - 2ms

20 - 5ms

21 - 10ms

22 - 20ms

23 - 50ms

24 - 100ms

25- 200ms

26 - 500ms

27 - 1s

28 - 2s

29 - 5s

30 - 10s

31 - 20s

32 - 50s

33 - 100s

34 - 200s

run_after_delay

Variable Type       ViReal64

This control selects the pretrig value ().

Default Value: 0.0

Valid Values are between -10*Range and 25*Range:

Range     Max Values    Min Values      Middle Values

200s   ->  5000          -2000           -1000

100s   ->  2500          -1000           -500

50s    ->  1250           -500           -250

20s    ->  500           -200            -100

10s    ->  250           -100            -50

5s     ->  125           -50             -25

2s     ->  50            -20             -10

1s     ->  25            -10             -5

500ms  ->  12500         -5000           -2500

200ms  ->  5000          -2000           -1000

100ms  ->  25*100        -10*100         -500

50ms   ->  25*50         -10*50          -250

20ms   ->  25*20         -10*20          -100

10ms   ->  25*10         -10*10          -50

5ms    ->  25*5          -10*5           -25

2ms    ->  25*2          -10*2           -10

1ms    ->  25*1          -10*1           -5

500us  ->  25*500        -500*10         -2500

200us  ->  25*200        -10*200         -1000

100us  ->  25*100        -10*100         -500

50us   ->  25*50         -10*50          -250

20us   ->  25*20         -10*20          -100

10us   ->  25*10         -10*10          -50

5us    ->  25*5          -10*5           -25

2us    ->  25*2          -10*2           -10

1us    ->  25*1          -10*1           -5

500ns  ->  25*500        -500*10         -2500

200ns  ->  25*200        -10*200         -1000

100ns  ->  25*100        -10*100         -500

50ns   ->  25*100        -10*100         -500

20ns   ->  25*100        -10*100         -500

10ns   ->  25*100        -10*100         -500

5ns    ->  25*100        -10*100         -500

2ns    ->  25*100        -10*100         -500

1ns    ->  25*100        -10*100         -500

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Size) out of range.

BFFC0003  Parameter 3 (Pretrig) out of range.

BFFC0004  Parameter 4 (Delay) out of range.

BFFC0005  Parameter 5 (RangeA) out of range.

BFFC0006  Parameter 6 (DelayA) out of range.

BFFC0007  Parameter 7 (RangeB) out of range.

BFFC0008  Parameter 8 (DelayB) out of range.

BFFC0803  Error scanning response.

BFFC0900  Instrument specific error.

BFFC0907  Parameter 7 (RangeB) is inconsistent.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_init

ViStatus mtx1054B_init (ViRsrc ressourceName, ViBoolean IDQuery,

ViPSession instrumentHandle);

Purpose

This function performs the following initialization actions:

- Opens a session to the Default Resource Manager resource and a session

to the specified device using the interface and address specified in the

Resource_Name control.

- Performs an identification query on the Instrument.

- Sends initialization commands to the instrument that set any necessary

programmatic variables such as Headers Off, Short Command form, and Data

Transfer Binary to the state necessary for the operation of the

instrument driver.

- Returns an Instrument Handle which is used to differentiate between

different sessions of this instrument driver.

- Each time this function is invoked a Unique Session is opened.  It is

possible to have more than one session open for the same resource.

Parameter List

ressourceName

Variable Type       ViRsrc

This control specifies the interface and address of the device that

is to be initialized (Instrument Descriptor). The exact grammar to be

used in this control is shown in the note below.

Default Value:  "USB"

Notes:

(1) Based on the Instrument Descriptor, this operation establishes a

communication session with a device.  The grammar for the Instrument

Descriptor is shown below.  Optional parameters are shown in square

brackets ([]).

Interface   Grammar

------------------------------------------------------

TCPIP       TCPIP[board]::host address::23::SOCKET

USB         USB

Example with TCPIP Parameters :

- Board = 0

- Port = 23

- IP address of oscilloscope = 192.168.0.100

"TCPIP0::192.168.0.100::23::SOCKET"

The default value for optional parameters are shown below.

Optional Parameter          Default Value

-----------------------------------------

board                       0

host address                IP address

IDQuery

Variable Type       ViBoolean

This control specifies if an ID Query is sent to the instrument

during the initialization procedure.

Valid Range:

VI_OFF (0) - Skip Query

VI_ON  (1) - Do Query (Default Value)

Notes:

(1) Under normal circumstances the ID Query ensures that the

instrument initialized is the type supported by this driver. However

circumstances may arise where it is undesirable to send an ID Query

to the instrument.  In those cases; set this control to "Skip Query"

and this function will initialize the selected interface, without

doing an ID Query.

instrumentHandle

Variable Type       ViSession (passed by reference)

This control returns an Instrument Handle that is used in all

subsequent function calls to differentiate between different sessions

of this instrument driver.

Notes:

(1) Each time this function is invoked a Unique Session is opened.

It is possible to have more than one session open for the same

resource.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (ID Query) out of range.

BFFC0003  Parameter 3 (Reset Device) out of range.

BFFC0011  Instrument returned invalid response to ID Query

BFFC0803  Error Scanning Response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_readChToArr

ViStatus mtx1054B_readChToArr (ViSession instrumentHandle,

ViInt16 source,

ViReal64 _VI_FAR waveformArray[],

ViPInt32 number_of_samples,

ViPReal64 xStart, ViPReal64 xIncrement);

Purpose

This function reads the specified channel from the instrument and places

it in an array.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

source

Variable Type       ViInt16

This control selects the source channel.

Valid Values:

0 - CH1  (Default Value)

1 - CH2

2 - CH3

3 - CH4

waveformArray

Variable Type       ViReal64[]

This function specifies the name of array in which to place the

waveform data.

Notes:

(1) The Real Array must be large enough to hold the requested

points.His numberr of elements is 50000.

number_of_samples

Variable Type       ViInt32 (passed by reference)

This function returns the number of valid points read from the

selected channel.

xStart

Variable Type       ViReal64 (passed by reference)

This function returns the time of the first data point in seconds

(with respect to the trigger point).

xIncrement

Variable Type       ViReal64 (passed by reference)

This function returns the incrementing value in seconds for the X

axis.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Source) out of range.

BFFC0003  Parameter 3 (Interpolation) out of range.

BFFC0803  Error scanning response.

BFFC0900  Instrument specific error.

BFFC090C  Channel is not active.

BFFC0910  Invalid file content.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_readChToFile

ViStatus mtx1054B_readChToFile (ViSession instrumentHandle,

ViInt16 source, ViString fileName,

ViBoolean overwriteFile);

Purpose

This function reads a channel from the instrument and stores it as a

file.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

source

Variable Type       ViInt16

This control selects the source channel.

Valid Values:

0 - CH1  (Default Value)

1 - CH2

2 - CH3

3 - CH4

fileName

Variable Type       ViString

This function specifies the name of the wave data file.

Default Value: ""

Notes:

(1) When storing a file to Computer, precede all backslashes with an

additional backslash, e.g. if the file is in the \cvi\instr

directory, enter the filename as "\\cvi\\instr\\anyfile".

(2) File name must be 15-character long max plus ".TRC" extension.The

extention must be in capital letter.

If not, an error is generated :0xBFFC0004

(3) If the target file already exists and overwrite is not selected,

an error is generated : 0xBFFC090E

overwriteFile

Variable Type       ViBoolean

This function specifies whether or not to write over the specified

file if it already exists.

Valid Range:

VI_OFF (0) - No  (Default Value)

VI_ON  (1) - Yes

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Source) out of range.

BFFC0003  Parameter 3 (Drive) out of range.

BFFC0004  Parameter 4 (Filename) out of specifications.

BFFC0005  Parameter 5 (Overwrite File?) out of range.

BFFC0801  Error writing to file.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFC090C  Channel is not active.

BFFC090E  File already exists.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_readCursors

ViStatus mtx1054B_readCursors (ViSession instrumentHandle,

ViInt16 reference, ViPReal64 t1_s,

ViPReal64 t2_s, ViPReal64 dT_s,

ViPReal64 tphase_s, ViPReal64 v1_V,

ViPReal64 v2_V, ViPReal64 dV_V,

ViPReal64 vphase_V);

Purpose

This function reads cursors positions.

All measured quantities are in their basic units.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

reference

Variable Type       ViInt16

This control selects the reference channel for the cursors positions.

Valid Values:

0 - CH1  (Default Value)

1 - CH2

2 - CH3

3 - CH4

t1_s

Variable Type       ViReal64 (passed by reference)

This control outputs the position of the vertical cursor #1.

t2_s

Variable Type       ViReal64 (passed by reference)

This control outputs the position of the vertical cursor #2.

dT_s

Variable Type       ViReal64 (passed by reference)

This control outpus the difference between vertical cursors.

tphase_s

Variable Type       ViReal64 (passed by reference)

This control outpus the difference between vertical cursors.

v1_V

Variable Type       ViReal64 (passed by reference)

This control outpus the position of the horizontal cursor #1.

v2_V

Variable Type       ViReal64 (passed by reference)

This control outpus the position of the horizontal cursor #2.

dV_V

Variable Type       ViReal64 (passed by reference)

This control outpus the difference between horizontal cursors.

vphase_V

Variable Type       ViReal64 (passed by reference)

This control outpus the difference between horizontal cursors.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Reference) out of range.

BFFC0003  Parameter 3 (T1) out of range.

BFFC0004  Parameter 4 (T2) out of range.

BFFC0005  Parameter 5 (dT) out of range.

BFFC0006  Parameter 6 (V1) out of range.

BFFC0007  Parameter 7 (V2) out of range.

BFFC0008  Parameter 8 (dV) out of range.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_readFileToArr

ViStatus mtx1054B_readFileToArr (ViSession instrumentHandle,

ViString fileName,

ViReal64 _VI_FAR waveformArray[],

ViPInt32 number_of_samples,

ViPReal64 xStart,

ViPReal64 xIncrement);

Purpose

This function reads a stored waveform file and places the data in an

array.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

fileName

Variable Type       ViString

This function specifies the name of the wave data file.

Default Value: ""

Notes:

(1) When reading a file from Computer, precede all backslashes with

an additional backslash, e.g. if the file is in the \cvi\instr

directory, enter the filename as "\\cvi\\instr\\anyfile".

(2) File name must be 15-character long max + ".TRC" extension.

The extention must be in capital letter.

If not an error is generated :0xBFFC0003

(3) If file does not exist an error is generated : 0xBFFC090D

waveformArray

Variable Type       ViReal64[]

This function specifies the name of array in which to place the

waveform data.

Notes:

(1) The Real Array must be large enough to hold the requested

points.His numberr of elements is 50000.

number_of_samples

Variable Type       ViInt32 (passed by reference)

This function returns the number of valid points read from the

selected file.

xStart

Variable Type       ViReal64 (passed by reference)

This function returns the time of the first data point in seconds

(with respect to the trigger point).

xIncrement

Variable Type       ViReal64 (passed by reference)

This function returns the incrementing value in seconds for the X

axis.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Drive) out of range.

BFFC0003  Parameter 3 (Filename) out of specifications.

BFFC0004  Parameter 4 (Interpolation) out of range.

BFFC0800  Error opening file.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFC090D  File not found.

BFFC0910  Invalid file content.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_readInstrData

ViStatus mtx1054B_readInstrData (ViSession instrumentHandle,

ViInt32 numberBytesToRead,

ViChar _VI_FAR readBuffer[],

ViPInt32 numBytesRead);

Purpose

This function reads data from the instrument's output buffer and returns

it to the specified variable in memory.

Notes:

(1) Because the instrument may return both numeric and text data in

response to queries this function returns the data in string format.

(2) If valid data is not available at the instrument's output buffer when

this function is called the instrument will hang up and the function will

not return until it times out.  If the time-out is disabled this function

will hang indefinitely and it may be necessary to reboot the computer to

break out.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

numberBytesToRead

Variable Type       ViInt32

The number of bytes specified by this control should be greater than

or equal to the number of bytes which are to be read from the

instrument. If the actual number of bytes to be read is greater than

the number this control specifies then multiple reads will be

required to empty the instrument's output buffer.

If the instrument's output buffer is not emptied the instrument may

return invalid data when future reads are performed.

Default Value:  50 (See NOTE 2)

Notes:

(1) If NO DATA is available at the instrument's output buffer when

this function is called the instrument will hang up and the function

will not return until it times out.  If the time-out is disabled,

this function will hang indefinitely and it may be necessary to

reboot the computer.

(2) If the number of bytes expected is greater than 50 the value of

this control may be increased. If the actual number of bytes read is

larger than the declared size of the read buffer a run-time error

will be generated.

readBuffer

Variable Type       ViChar[]

The incoming data from the instrument is placed into this variable.

Notes:

(1) Because the instrument may return both numeric and text data in

response to queries this function returns the data in string format.

(2) This function does not overwrite any old data left in the string

variable from the last time the function was called. String data in

LabWindows/CVI is terminated with an ASCII null(0x0) and string

manipulation functions will only recognize data before the ASCII

null.

(3) The declared size of the string variable must be greater than the

actual number of bytes read from the instrument if it is not a

run-time error will be generated.

numBytesRead

Variable Type       ViInt32 (passed by reference)

This variable contains the actual number of bytes read from the

instrument. This is the value which is returned by the read function.

Notes:

(1) If the actual number of bytes read is less than the number of

bytes specified in the Number Bytes To Read control then the output

buffer has probably been emptied. If the read function fails and the

number of bytes read is 0, the most probable cause for the failure is

there was no data available at the instrument's output buffer.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_readMeasurement

ViStatus mtx1054B_readMeasurement (ViSession instrumentHandle,

ViInt16 measurement,

ViInt16 reference,

ViPReal64 reading);

Purpose

This function forces the instrument to perform selected measurement and

returns the result.

All measured quantities are in their basic units.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

measurement

Variable Type       ViInt16

This control specifies the measurement function.

Valid Values:

0  - Minimum Voltage     [V]

1  - Maximum Voltage     [V]

2  - Peak to Peak        [V] (Default Value)

3  - Low Voltage         [V]

4  - High Voltage        [V]

5  - Amplitude           [V]

6  - Voltage RMS Voltage [V]

7  - Average Voltage     [V]

8  - Rise Overshoot      [%]

9  - Fall Overshoot      [%]

10 - Rise Time           [s]

11 - Fall Time           [s]

12 - Pos Pulse Width     [s]

13 - Neg Pulse Width     [s]

14 - Period               [s]

15 - Frequency           [Hz]

16 - Dutycycle           [%]

17 - N

18 _ Phase

reference

Variable Type       ViInt16

This control selects the reference channel for the specified

measurement.

Valid Values:

0 - CH1  (Default Value)

1 - CH2

2 - CH3

3 - CH4

reading

Variable Type       ViReal64 (passed by reference)

This control outputs the result of the measurement.

All measured quantities are output in their basic units.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Measurement) out of range.

BFFC0003  Parameter 3 (Reference) out of range.

BFFC0004  Parameter 4 (Reading) out of range.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_reset

ViStatus mtx1054B_reset (ViSession instrumentHandle);

Purpose

This function resets the instrument to a known state.

See the Digiscope 2000 Operating Instructions manual for more details.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_revisionQuery

ViStatus mtx1054B_revisionQuery (ViSession instrumentHandle,

ViChar _VI_FAR instrumentDriverRevision[],

ViChar _VI_FAR firmwareRevision[]);

Purpose

This function returns the revision numbers of the instrument driver and

instrument firmware, and tells the user what firmware the driver is

compatible with, for example:

This instrument driver's Revision Number is "Rev 1.0, 04/2005, CVI 6.0"

and is compatible with Instrument Firmware Revision "1.6".

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

instrumentDriverRevision

Variable Type       ViChar[]

This control returns the Instrument Driver Software Revision.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

firmwareRevision

Variable Type       ViChar[]

This control returns the Instrument Firmware Revision.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_runAcquisition

ViStatus mtx1054B_runAcquisition (ViSession instrumentHandle,

ViBoolean acquisition);

Purpose

This function controls the acquisition system of the instrument.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

acquisition

Variable Type       ViBoolean

This control valid the acquisition system of the instrument.

Valid Values:

VI_OFF (0) - Stop

VI_ON  (1) - Run   (Default Value)

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Mode) out of range.

BFFC0003  Parameter 3 (Wait?) out of range.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_runAutoset

ViStatus mtx1054B_runAutoset (ViSession instrumentHandle);

Purpose

This function performs an autoset of the instrument.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0803  Error scanning response.

BFFC0900  Instrument specific error.

BFFC090F  No channel is enabled.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_saveSetup

ViStatus mtx1054B_saveSetup (ViSession instrumentHandle, ViBoolean mode,

ViString fileName,

ViBoolean overwriteFile);

Purpose

This function save or recall set-up to or from file.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

mode

Variable Type       ViBoolean

This control selects between Save and Recall mode.

Valid Range:

VI_OFF (0) - Recall

VI_ON  (1) - Save  (Default Value)

fileName

Variable Type       ViString

This function specifies the name of the setup data file.

Default Value: ""

Notes:

(1) When reading/writing a file from/to Computer, precede all

backslashes with an additional backslash, e.g. if the file is in the

\cvi\instr directory, enter the filename as "\\cvi\\instr\\anyfile".

(2) File name must be 8-character long max + ".CFG" extension.

The extention must be in capital letter.

If not an error is generated :0xBFFC0004

(3) When saving a setup : if the target file already exists and

overwrite is not selected an error is generated : 0xBFFC090E

(4) When recalling a setup : if file does not exist an error is

generated : 0xBFFC090D

overwriteFile

Variable Type       ViBoolean

This function specifies whether or not to write over the specified

file if it already exists on disk.

Valid Range:

VI_OFF (0) - No  (Default Value)

VI_ON  (1) - Yes

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Mode) out of range.

BFFC0003  Parameter 3 (Drive) out of range.

BFFC0004  Parameter 4 (Filename) out of specifications.

BFFC0005  Parameter 5 (Overwrite File?) out of range.

BFFC0800  Error opening file.

BFFC0801  Error writing to file.

BFFC0803  Error Scanning Response

BFFC0900  Instrument specific error.

BFFC090D  File not found.

BFFC090E  File already exists.

BFFC0910  Invalid file content.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_selfTest

ViStatus mtx1054B_selfTest (ViSession instrumentHandle,

ViPInt16 selfTestResult,

ViChar _VI_FAR selfTestMessage[]);

Purpose

This function runs the instrument's self test routine and returns the

test result(s).

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

selfTestResult

Variable Type       ViInt16 (passed by reference)

This control contains the value returned from the instrument self

test.  Zero means success.  For any other code, see the device's

operator's manual.

selfTestMessage

Variable Type       ViChar[]

This control contains the string returned from the self test. See the

device's operation manual for an explanation of the string's

contents.

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0803  Error scanning response.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_setCountTrigger

ViStatus mtx1054B_setCountTrigger (ViSession instrumentHandle,

ViInt16 auxiliarySource,

ViBoolean edge, ViInt16 coupling,

ViInt16 noiseRejection,

ViReal64 level_volts, ViInt32 events,

ViReal64 holdoff_seconds);

Purpose

This function configures the count trigger parameters of the instrument.

The following settings are available :

- Auxiliary Source

- Edge

- Coupling

- Noise Rejection

- Level

- Events

- Holdoff

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

auxiliarySource

Variable Type       ViInt16

This control selects the counter source.

Valid Values:

1 - CH1  (Default Value)

2 - CH2

3 - CH3

4 - CH4

edge

Variable Type       ViBoolean

This control sets the slope of the specified channel.

Valid Values:

VI_ON  (1) - Pos (Default Value)

VI_OFF (0) - Neg

coupling

Variable Type       ViInt16

This control selects the coupling mode of the specified channel.

Valid Values:

0 - AC (Default Value)

1 - DC

2 - LF Rej

3 - HF Rej

noiseRejection

Variable Type       ViInt16

This control sets the noise rejection of the specified channel.

Valid Values:

0 - OFF  (Default Value)

3 - ON

level_volts

Variable Type       ViReal64

This control sets level of trigger for the specified channel.

Valid Range:

-7.5 * vertSense (vertSense in Volts/div)  to  7.5 * vertSense

(vertSense in Volts/div)

Default Value: 0.0 V

events

Variable Type       ViInt32

This control sets the number of events to count.

Valid Range:

3 to 16 384

Default Value: 3

holdoff_seconds

Variable Type       ViReal64

This control sets the holdoff value.

Valid Range:

40.0 ns to 30s

Default Value: 60.0 ns

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Trigger source) out of range.

BFFC0003  Parameter 3 (Qualifier) out of range.

BFFC0004  Parameter 4 (Counter) out of range.

BFFC0005  Parameter 5 (Events) out of range.

BFFC0905  Parameter 5 (Events) is inconsistent.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_setCursors

ViStatus mtx1054B_setCursors (ViSession instrumentHandle,

ViInt16 cursors, ViInt16 reference,

ViBoolean unattached_cursors,

ViReal64 t1_s, ViReal64 t2_s,

ViReal64 tphase_s);

Purpose

This function sets horizontal cursors.

The following settings are available :

- Cursors

- Reference

- Unattached cursors

- T1

- T2

- TPhase

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

cursors

Variable Type       ViInt16

This control selects the cursors mode.

Valid Values:

0 - OFF  (Default Value)

1 - ON

2 - ON+PHASE

reference

Variable Type       ViInt16

This control selects the reference.

Valid Values:

0 - CH1  (Default Value)

1 - CH2

2 - CH3

3 - CH4

unattached_cursors

Variable Type       ViBoolean

This control selects the unattached cursors.

Valid Values:

VI_OFF (0) - Off

VI_ON  (1) - On   (Default Value)

t1_s

Variable Type       ViReal64

This control sets the position of the vertical cursor #1.

Valid Range:

+/- 100 * horizSense (horizSense in s/div)

t2_s

Variable Type       ViReal64

This control sets the position of the vertical cursor #2.

Valid Range:

+/- 100 * horizSense (horizSense in s/div)

tphase_s

Variable Type       ViReal64

This control sets the position of the vertical cursor #3 (phase).

Valid Range:

+/- 100 * horizSense (horizSense in s/div)

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Clear Cursors) out of range.

BFFC0003  Parameter 3 (Reference) out of range.

BFFC0004  Parameter 4 (T1) out of range.

BFFC0005  Parameter 5 (T2) out of range.

BFFC0006  Parameter 6 (V1) out of range.

BFFC0007  Parameter 7 (V2) out of range.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_setDelayedTrigger

ViStatus mtx1054B_setDelayedTrigger (ViSession instrumentHandle,

ViInt16 auxiliarySource,

ViBoolean edge, ViInt16 coupling,

ViInt16 noiseRejection,

ViReal64 level_volts,

ViReal64 delay,

ViReal64 holdoff_seconds);

Purpose

This function configures the delayed trigger parameters of the

instrument.

The following settings are available :

- Auxiliary Source

- Edge

- Coupling

- Noise Rejection

- Level

- Delay

- Holdoff

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

auxiliarySource

Variable Type       ViInt16

This control selects the auxilary source.

Valid Values:

1 - CH1  (Default Value)

2 - CH2

3 - CH3

4 - CH4

edge

Variable Type       ViBoolean

This control sets the slope of the specified channel.

Valid Values:

VI_ON  (1) - Pos (Default Value)

VI_OFF (0) - Neg

coupling

Variable Type       ViInt16

This control selects the coupling mode of the specified channel.

Valid Values:

0 - AC (Default Value)

1 - DC

2 - LF Rej

3 - HF Rej

noiseRejection

Variable Type       ViInt16

This control sets the noise rejection of the specified channel.

Valid Values:

0 - OFF  (Default Value)

3 - ON

level_volts

Variable Type       ViReal64

This control sets level of trigger for the specified channel.

Valid Range:

-7.5 * vertSense (vertSense in Volts/div)  to  7.5 * vertSense

(vertSense in Volts/div)

Default Value: 0.0 V

delay

Variable Type       ViReal64

This control sets the delay value in seconds.

Valid Range:

20 ns  to  10.5 s

Default Value: 20 ns

holdoff_seconds

Variable Type       ViReal64

This control sets the holdoff value.

Valid Range:

40.0 ns to 10.5s

Default Value: 40.0 ns

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Trigger source) out of range.

BFFC0003  Parameter 3 (Qualifier) out of range.

BFFC0004  Parameter 4 (Delay) out of range.

BFFC0803  Error scanning response.

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_setGeneralTrigger

ViStatus mtx1054B_setGeneralTrigger (ViSession instrumentHandle,

ViInt16 triggerSource,

ViBoolean slope, ViInt16 coupling,

ViBoolean noiseRejection,

ViInt16 type, ViReal64 level_volts,

ViReal64 holdoff_seconds);

Purpose

This function configures the general trigger parameters of the

instrument.

The following settings are available :

- Trigger Source

- Slope

- Coupling

- Noise Rejection

- Type

- Level

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

triggerSource

Variable Type       ViInt16

This control selects the trigger source to configure.

Valid Values:

0 - CH1  (Default Value)

1 - CH2

2 - CH3

3 - CH4

4 - LINE

slope

Variable Type       ViBoolean

This control sets the slope of the specified channel.

Valid Values:

VI_ON  (1) - Up (Default Value)

VI_OFF (0) - Down

coupling

Variable Type       ViInt16

This control selects the coupling mode of the specified channel.

Valid Values:

0 - AC (Default Value)

1 - DC

2 - LF Rej

3 - HF Rej

4 _ TV-Field

5 - TV-line

noiseRejection

Variable Type       ViBoolean

This control sets the noise rejection of the specified channel.

Valid Values:

VI_OFF (0) - Off  (Default Value)

VI_ON  (1) - On

type

Variable Type       ViInt16

This control selects the type of trigger.

Valid Values:

0 - Triggered

1 - Auto (Default Value)

2 - Single

level_volts

Variable Type       ViReal64

This control sets level of trigger for the specified channel.

Valid Range:

-7.5 * vertSense (vertSense in Volts/div)  to  7.5 * vertSense

(vertSense in Volts/div)

Default Value: 0.0 V

holdoff_seconds

Variable Type       ViReal64

This control sets the holdoff value.

Valid Range:

40.0 ns to 10.5s

Default Value: 40.0 ns

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Channel) out of range.

BFFC0003  Parameter 3 (Slope) out of range.

BFFC0004  Parameter 4 (Coupling) out of range.

BFFC0005  Parameter 5 (HF Rejection) out of range.

BFFC0006  Parameter 6 (Noise Rejection) out of range.

BFFC0007  Parameter 7 (Type) out of range.

BFFC0008  Parameter 8 (Level) out of range.

BFFC0803  Error scanning response.

BFFC0809  Parameter 9 (Holdoff) out of range.

BFFC0900  Instrument specific error.

BFFC0905  Parameter 5 (HF Rejection) is inconsistent.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_setOptionDisplay

ViStatus mtx1054B_setOptionDisplay (ViSession instrumentHandle,

ViInt16 deviceMode);

Purpose

This function configures the display options of the instrument.

The following settings are available :

- Grid

- Divice Mode

- Advanced mode

- Language

- Standby

- Screen

- Brightness

- Contrast

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

deviceMode

Variable Type       ViInt16

This control selects the principal mode.

Valid Values:

0 - Scope   (Default Value)

1 - Analyser

2 - Recorder

3 - SPO

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Grid) out of range.

BFFC0003  Parameter 3 (Color) out of range.

BFFC0004  Parameter 4 (Screen) out of range.

BFFC0005  Parameter 5 (Menus) out of range.

BFFC0006  Parameter 6 (Language) out of range.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_setPulsTrigger

ViStatus mtx1054B_setPulsTrigger (ViSession instrumentHandle,

ViInt16 triggerPulsate_type,

ViReal64 pulse);

Purpose

This function configures the pulsate trigger mode of the instrument.

The following settings are available :

- Trigger pulsate type

- Pulse

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

triggerPulsate_type

Variable Type       ViInt16

This control selects the trigger pulsate type.

Valid Values:

0 - Equate  (Default Value)

1 - Superior

2 - Inferior

pulse

Variable Type       ViReal64

This control sets the durate  of the pulse.

Valid Values:

10ns   to  10.5s

Default Value: 10ns

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Source) out of range.

BFFC0003  Parameter 3 (Window) out of range.

BFFC0004  Parameter 4 (Main Level) out of range.

BFFC0005  Parameter 5 (Auxiliary Level) out of range.

BFFC0903  Parameter 3 (Window) is inconsistent.

BFFC0803  Error scanning response.

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_setSPOOptions

ViStatus mtx1054B_setSPOOptions (ViSession instrumentHandle,

ViInt16 palette, ViInt16 timing);

Purpose

This function configures the waveform display of the instrument.

The following settings are available :

- Min/Max acquisition

- Average rate

- Mode

- Repetitive Signal

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

palette

Variable Type       ViInt16

This control sets the palette of colors.

Valid Values:

0 - Monochrome   (Default Value)

1 - Multicolor

timing

Variable Type       ViInt16

This control sets the timimg of persistance.

Valid Values:

0 - 100ms

1 - 200ms

3 - 500ms (Default Value)

4 - 1s

5 - 2s

6 - 5s

7 - 10s

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Mode) out of range.

BFFC0003  Parameter 3 (Average rate) out of range.

BFFC0004  Parameter 4 (Persistence) out of range.

BFFC0005  Parameter 5 (Interpolation) out of range.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_setVideoTrigger

ViStatus mtx1054B_setVideoTrigger (ViSession instrumentHandle,

ViBoolean standard,

ViBoolean polarity, ViInt16 line,

ViReal64 holdoff_seconds);

Purpose

This function configures the video trigger mode of the instrument.

The following settings are available :

- Standard

- Polarity

- Line

- Holdoff

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

standard

Variable Type       ViBoolean

This control selects the standard.

Valid Values:

VI_OFF (0) - 525

VI_ON  (1) - 625  (Default Value)

polarity

Variable Type       ViBoolean

This control selects the polarity.

Valid Values:

VI_OFF (0) - Pos (+)  (Default Value)

VI_ON  (1) - Neg (-)

line

Variable Type       ViInt16

This control selects the number of the line.

Default Value: 1

holdoff_seconds

Variable Type       ViReal64

This control sets the holdoff value.

Valid Range:

40.0 ns to 30s

Default Value: 60.0 ns

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Trigger Source) out of range.

BFFC0003  Parameter 3 (Standard) out of range.

BFFC0004  Parameter 4 (Polarity) out of range.

BFFC0005  Parameter 5 (Field) out of range.

BFFC0006  Parameter 6 (line) out of range.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFC0906  Parameter 6 (Line) is inconsistent.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_setWaveformDisplay

ViStatus mtx1054B_setWaveformDisplay (ViSession instrumentHandle,

ViInt16 minMax,

ViInt16 average_rate,

ViInt16 displayMode,

ViBoolean repetitiveSignal);

Purpose

This function configures the waveform display of the instrument.

The following settings are available :

- Min/Max acquisition

- Average rate

- Mode

- Repetitive Signal

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

minMax

Variable Type       ViInt16

This control Valids the Min/Max Acquisition.

Valid Values:

0 - Normal_OFF   (Default Value)

1 - Envelope_ ON

average_rate

Variable Type       ViInt16

This control sets the average rate.

Valid Values:

0 - No averaging (Default Value)

1 - 2

2 - 4

3 - 16

4 - 64

displayMode

Variable Type       ViInt16

This control selects the display mode.

Valid Values:

0 - Vector   (Default Value)

1 - Envelope

repetitiveSignal

Variable Type       ViBoolean

This control selects the repetitive signal.

Valid Values:

VI_OFF (0) - Off

VI_ON  (1) - On   (Default Value)

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Mode) out of range.

BFFC0003  Parameter 3 (Average rate) out of range.

BFFC0004  Parameter 4 (Persistence) out of range.

BFFC0005  Parameter 5 (Interpolation) out of range.

BFFC0803  Error scanning response

BFFC0900  Instrument specific error.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_verticalSetup

ViStatus mtx1054B_verticalSetup (ViSession instrumentHandle,

ViInt16 input, ViBoolean state,

ViInt16 coupling, ViInt16 probe,

ViInt16 voltDiv, ViReal64 offset_volts,

ViReal64 bandwidth);

Purpose

This function configures the selected input channel of the instrument.

The following settings are available :

- Channel

- State

- Coupling

- Probe

- Channel inversion

- Range

- Offset

- Bandwidth limitation

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

input

Variable Type       ViInt16

This control selects the channel to configure.

Valid Values:

1 - CH1  (Default Value)

2 - CH2

3 - CH3

4 - CH4

state

Variable Type       ViBoolean

This control selects the state of the specified channel.

Valid Values:

VI_ON  (1) - On  (Default Value)

VI_OFF (0) - Off

coupling

Variable Type       ViInt16

This control selects the coupling of the specified channel.

Valid Values:

0 - AC

1 - DC  (Default Value)

2 - GND

probe

Variable Type       ViInt16

This control selects the probe of the specified channel.

Valid Values:

1 - x 1  (Default Value)

2 - x 10

3 - x 100

4 - x 1000

voltDiv

Variable Type       ViInt16

This control sets vertical sense for the specified channel.

Valid Values:

0  - 2.50mV/div

1  - 5.00mV/div

2  - 10.0mV/div

3  - 20.0mV/div

4  - 50.0mV/div

5  - 100mV/div

6  - 200mV/div

7  - 500mV/div

8  - 1V/div     (Default Value)

9  - 2V/div

10 - 5V/div

11 - 10V/div

12 - 20V/div

13 - 50V/div

14 - 100V/div

15 - 200V/div

16 - 500V/div

17 - 1kV/div

Note : This control value must be consistent with the probe value.

See the Digiscope 2000 Operating Instructions manual for more

details. If it is inconsistent an error is generated :

VI_ERROR_INCONSISTENT_PARAMETER7 (0xBFFC0907)

offset_volts

Variable Type       ViReal64

This control sets the signal offset of the specified channel.

Valid Ranges:

vertical sense       offset range

2.5mV/div              +/- 80mV

5mV/div              +/- 80mV

10mV/div             +/- 400mV

20mV/div             +/- 400mV

50mV/div             +/- 2V

100mV/div            +/- 2V

200mV/div            +/- 2.5V

500mV/div            +/- 12.5V

1V/div               +/- 12.5V

2V/div               +/- 64V

5V/div               +/- 64V

10V/div              +/- 64V

20V/div              +/- 640V

50V/div              +/- 640V

100V/div             +/- 640V

200V/div             +/- 6400V

500V/div             +/- 6400V

1kV/div              +/- 6400V

Default Value: 0.0

bandwidth

Variable Type       ViReal64

This control selects the state of the specified channel.

Valid Values:

VI_ON  (1)   (Default Value)

VI_OFF (0)

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFC0002  Parameter 2 (Input) out of range.

BFFC0003  Parameter 3 (State) out of range.

BFFC0004  Parameter 4 (Coupling) out of range.

BFFC0005  Parameter 5 (Probe) out of range.

BFFC0006  Parameter 6 (Polarity) out of range.

BFFC0007  Parameter 7 (Volts/div) out of range.

BFFC0008  Parameter 8 (Offset) out of range.

BFFC0803  Error scanning response.

BFFC0809  Parameter 9 (Bandwidth) out of range.

BFFC0900  Instrument specific error.

BFFC0907  Parameter 7 (Volts/div) is inconsistent.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

mtx1054B_writeInstrData

ViStatus mtx1054B_writeInstrData (ViSession instrumentHandle,

ViString writeBuffer);

Purpose

This function writes commands and queries to the instrument to modify

parameters and query device settings.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

writeBuffer

Variable Type       ViString

The user can use this control to send common commands and queries to

the instrument. This control can also be used to write any valid

command to the instrument.

Default Value:  "*CLS\r"

Notes:

(1) The command or query to be sent to the instrument may be a

literal enclosed in double quotes terminated by '\r' i.e. "*CLS\r" or

may be contained in a variable of type string.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.