OX860 Metrix

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for OX860 Metrix.

It contains functions for opening, configuring, taking measurement from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For GPIB instruments drivers:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

For VXI instruments drivers:

-  the instrument is installed in the VXI mainframe and you are

using one of the following controller options:

Embedded controller

AT-MXI

MC-MXI

GPIB-VXI

-  the logical address supplied to the initialize function must

match the logical address of the instrument.

-  if you are using GPIB-VXI, there must be at least one unused

device name in the handler.

For RS-232 instruments drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error Codes:

Error codes are returned as the return value of each instrument driver

function. A program should examine this value after each call to an instrument

driver function to determine if any error occurred. Possible error codes and

their meanings are listed with the corresponding instrument driver function.

________________________________________________________________________________

How To Use This Document:

This document is intended to be used as a programming reference manual.

It describes each function in the

OX860 Metrix

instrument. The functions are listed in alphabetical order with a

description of the function, C syntax of the function, a description of each

parameter, and possible error codes.

________________________________________________________________________________

Function Tree Layout:

OX860 Metrix                           Function Name:

Initialize                                  ox860_init

Instrument Setup                            ox860_setup

Configure

Autoset                                  ox860_autoset

Display settings                         ox860_display

Channel settings                         ox860_channel

Time base                                ox860_timebase

Trigger                                  ox860_trigger

Close                                       ox860_close

________________________________________________________________________________

OX860 Metrix

Instrument name: OX860 Metrix

Description: This instrument module provides

RS232 support for OX860.

The module contains the followings

functions:

(1) Initialize

Initialize the instrument module

with default values and lock the

front panel.

(2) Instrument setup

(3) Instrument Configuration

Class of functions for configuring

the following parameters:

* Autoset

* Display settings

* Channel settings

* Time Base

* Trigger

(4) Close

Close the instrument module and

unlock the front panel.

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

ox860_autoset

int ox860_autoset (void);

Purpose

Pannel name : Autoset

Description :

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  :  integer

Variable range :

0  :  Success

230  :  Error writing to instrument

232  :  Instrument not initialized

Others  :  See rs232err values

________________________________________________________________________________

ox860_channel

int ox860_channel (int channel, int coupling, int range, int invertCH2);

Purpose

Panel name  :   Channel settings

Description :   Sets the Channel , its

coupling and volts/division .

Sets the Invert CH2 .

Coupling :  AC DC GND

Volts/Div:  1  mV/Div  to 20 V/Div

with 1-2-5 sequences .

Invert CH2 : Off On

Parameter List

channel

Variable Type       int

Control label  : Channel number

Description    : Specifies the channel of the

oscilloscope to configure.

Variable type  : integer

Variable range :    0: CH1 (default value)

1: CH2

coupling

Variable Type       int

Control label  : Coupling

Description    : Set the coupling for the

selected channel.

Variable type  : integer

Variable range :     0 :  AC (default value)

1 :  DC

2 :  GND

range

Variable Type       int

Control label  : Range

Description    : Set the range (V/Div)

of the specified channel

Variable type  : integer

Variable range :    10   :   2  mV/Div

9   :   5  mV/Div

8   :   10 mV/Div

7   :   20 mV/Div

6   :   50 mV/Div

5   :   0.1 V/Div  (default value)

4   :   0.2 V/Div

3   :   0.5 V/Div

2   :   1   V/Div

1   :   2   V/Div

0   :   5   V/Div

invertCH2

Variable Type       int

Control label  : Invert CH2

Description    : Inverts Channel 2

Variable type  : integer

Variable range :    0 : Off  (default value)

1 : On

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  :  global integer

Variable range :

0  :  Success

-1  :  Channel value out of range

-2  :  Coupling value out of range

-3  :  Range value out of range

-4  :  Invert CH2 value out of range

230   :  Error writing to instrument

232   :  Instrument not initialized

Others :  See rs232err values

________________________________________________________________________________

ox860_close

int ox860_close (void);

Purpose

Pannel name : Close

Description   : Close the instrument module

and unlock the front panel.

NOTE: The instrument must be re-

initialized to use it again.

Return Value

Control label  : Error

Description    : Displays the result of the

function call.

Variable type  : integer

Variable range :

0 :    Success

221 :    Unable to close instrument

232 :    Instrument not initialized

Others  :    See rs232err values

________________________________________________________________________________

ox860_display

int ox860_display (int verticalMode, int BWL, int beamFinder);

Purpose

Panel name  :   Display settings

Description :   This panel is usefull to

configure the Vertical mode

and to control the Beam Find .

Vertical Mode : CH1 CH2 ALT CHOP ADD XY

Bwl           : Off On

Beam Find     : Off On

Parameter List

verticalMode

Variable Type       int

Control label  : Vertical Mode

Description    : Sets the vertical mode

Variable type  : integer

Variable range : 0 : CH1 (default value)

1 : CH2

2 : ALT

3 : CHOP

4 : ADD

5 : XY

BWL

Variable Type       int

Control label  : BWL

Description    : This control configures the

BWL mode : On or Off .

Variable type  : integer

Variable range :    0 : Off (default value)

1 : On

beamFinder

Variable Type       int

Control label  :    Beam Finder

Description    :    Set the Beam Finder

Variable type  :    integer

Variable range :    0 : Off (default value)

1 : On

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  :  integer

Variable range :

0  :       Success

-1  :       Vertical Mode out of

range

-2  :       Bwl out of range

-3  :       Beam Finder out of range

230   :  Error writing to instrument

232   :  Instrument not initialized

Others :  See rs232err values

________________________________________________________________________________

ox860_init

int ox860_init (int RS232portnumber);

Purpose

Panel name : Initialize

Description : Opens the instrument

with the following RS232

configuration :

9600 Bauds,8 bits,no parity

1 stop bit, XON/XOFF

and set the instrument to

default values.

Parameter List

RS232portnumber

Variable Type       int

Control label  : RS232 port number

Description    : Defines the RS232 port number

Variable type  : integer

Variable range : 1 to 8

Default value : 1

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  :  global integer

Variable range :

0   :  Success

-1   :  Invalid port number RS232

-2   :  Invalid Lock front panel number.

223   :  Instrument identification

query failed.

230   :  Error writing to instrument

Others   :  See rs232err

________________________________________________________________________________

ox860_setup

int ox860_setup (int setup, char filename[]);

Purpose

Panel name  :   Instrument Setup

Description :   This panel is usefull to

Save or Recall the instrument

Setup to or from a file .

This file is in ASCII format and

contains the values of the

functions listed below :

Function                  Parameter         EOT

Vertical Mode     : 0x64   0x30 to 0x35      0x04

Sensibility CH1   : 0x60   0x30 to 0x3D      0x04

Sensibility CH2   : 0x61   0x30 to 0x3D      0x04

Coupling    CH1   : 0x6a   0x30 to 0x32      0x04

Coupling    CH2   : 0x6c   0x30 to 0x32      0x04

Invert CH2        : 0x6e   0x30 to 0x31      0x04

Time Base Range A : 0x62   0x30 to 0x43      0x04

Time Base x10     : 0x71   0x30 to 0x31      0x04

Source   Trigger  : 0x68   0x30 to 0x34      0x04

Auto     Trigger  : 0x73   0x30 to 0x31      0x04

Coupling Trigger  : 0x69   0x30 to 0x35      0x04

Peak     Trigger  : 0x70   0x30 to 0x31      0x04

Slope    Trigger  : 0x72   0x30 to 0x31      0x04

Delayed  Trigger  : 0x65   0x30 to 0x32      0x04

Write Config      : 0x77   0x30              0x04

Time Base Range B : 0x63   0x30 to 0x43      0x04

Bwl               : 0x6f   0x30 to 0x31      0x04

B trig            : 0x66   0x30 to 0x31      0x04

Trace sep         : 0x67   0x30 to 0x3f      0x04

Parameter List

setup

Variable Type       int

Control label  :  Configuration

Description    :  Selects the Save or Recall

Instrument Setup function .

Variable type  : integer

Variable range :     0 : Save (default value)

1 : Recall

filename

Variable Type       char []

Control label  : File name

Description    : Defines the File name which

contains the Instrument

Setup.

Variable type  : string

Variable range : any valid string

(use \\\ with Quick C in case

of complete pathname :

Example  : "c:\\\lw\\\programs\\\example")

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  :  global integer

Variable range :

0  :  Success

-1  :  Setup value out of range

-2  :  File doesn't exist

230   :  Error writing to instrument

231   :  Error reading from instrument

232   :  Instrument not initialized

Others :  See rs232err values

________________________________________________________________________________

ox860_timebase

int ox860_timebase (int x10, int rangeA, int delay, int btrig, int rangeB,

int tracesep);

Purpose

Panel name  :   Time base

Description :   This panel is usefull to set

the Time base parameters of

the oscilloscope :

x10       :  Off On

Range A   :  100 ms/Div to 50nS/Div

with 1-2-5 sequences

Delayed TB:  Off Search Delayed

B trig    :  Off On

Range B   :  100 ms/Div to 50nS/Div

with 1-2-5 sequences

Trace sep :  O step to 15 steps

Parameter List

x10

Variable Type       int

Control label  : x10

Description    : This control configures the

horizontal x10 expanded mode

On or Off .

Variable type  : integer

Variable range :    0 : Off  (default value)

1 : On

rangeA

Variable Type       int

Control label  : Range

Description    : Set the timebase Range A of

the oscilloscope .

Variable type  : integer

Variable range :    0   :   100 mS

1   :    50 mS

2   :    20 mS

3   :    10 mS

4   :     5 mS

5   :     2 mS

6   :     1 mS

7   :   500 S

8   :   200 S

9   :   100 S

10  :    50 S

11  :    20 S (default value)

12  :    10 S

13  :     5 S

14  :     2 S

15  :     1 S

16  :   0.5 S

17  :   0.2 S

18  :   0.1 S

19  :    50 nS

delay

Variable Type       int

Control label  : Delay

Description    : This control configures the

delayed time base state.

Variable type  : integer

Variable range :    0 : Off  (default value)

1 : Alterned

2 : Delay

btrig

Variable Type       int

Control label  : B trig

Description    : Set the B trig mode

Off or On .

Variable type  : integer

Variable range :   0 : Off (default value)

1 : On

rangeB

Variable Type       int

Control label  : Range

Description    : Set the timebase Range B of

the oscilloscope .

Variable type  : integer

Variable range :    0   :   100 mS

1   :    50 mS

2   :    20 mS

3   :    10 mS

4   :     5 mS

5   :     2 mS

6   :     1 mS

7   :   500 S

8   :   200 S

9   :   100 S

10  :    50 S

11  :    20 S  (default value)

12  :    10 S

13  :     5 S

14  :     2 S

15  :     1 S

16  :   0.5 S

17  :   0.2 S

18  :   0.1 S

19  :    50 nS

tracesep

Variable Type       int

Control label  : Trace sep

Description    : Set the number of step for

the Trace Sep mode.

Variable type  : integer

Variable range : 0 : 0 step  (default value)

1 : 1 step

2 : 2 steps

3 : 3 steps

4 : 4 steps

5 : 5 steps

6 : 6 steps

7 : 7 steps

8 : 8 steps

9 : 9 steps

10 : 10 steps

11 : 11 steps

12 : 12 steps

13 : 13 steps

14 : 14 steps

15 : 15 steps

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  : integer

Variable range :

0   :    Success

-1   :    x10 value out of range

-2   :    Range A value out of range

-3   :    Delay value out of range

-4   :    B trig value out of range

-5   :    Range B value out of range

-6   :    Trace sep value out of range

-7   :    Vertical Mode = XY

-8   :    Range B > Range A

230  :  Error writing to instrument

232  :  Instrument not initialized

Others  :  See rs232err values

________________________________________________________________________________

ox860_trigger

int ox860_trigger (int ptp, int slope, int auto, int source, int coupling);

Purpose

Panel name  :   Trigger

Description :   This panel is usefull to

configure the following

trigger parameters:

- Peak to Peak : on/off

- Slope : + / -

- Auto  : on/off

- Source : CH1,CH2,LINE,

ALT,EXT

- Coupling : AC,DC,LFR,

HFR,TVH,TVV

Parameter List

ptp

Variable Type       int

Control label  : Ptp

Description    : Set the trigger Peak to Peak

mode Off or On .

Variable type  : integer

Variable range :    0 : Off

1 : On  (default value)

slope

Variable Type       int

Control label  : Slope

Description    : Set the trigger slope

Positive or negative

Variable type  : integer

Variable range :    0 : Positive  (default value)

1 : Negative

auto

Variable Type       int

Control label  : Auto

Description    : Sets the trigger Auto mode

Off or On .

Variable type  : integer

Variable range :    0 : Off

1 : On  (default value)

source

Variable Type       int

Control label  : Source

Description    : Sets the trigger Source .

Variable type  : integer

Variable range : 0 : CH1 (default value)

1 : CH2

2 : ALT

3 : LINE

4 : EXT

When Trigger Source LINE is configured, Trigger

Coupling is disabled.

coupling

Variable Type       int

Control label  : Coupling

Description    : Sets the trigger Coupling .

Variable type  : integer

Variable range : 0 : DC  (default value)

1 : AC

2 : LFR

3 : HFR

4 : TVH

5 : TVV

When Trigger Source LINE is configured, Trigger

Coupling is disabled.

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  : integer

Variable range :

0  :     Success

-1  :     Ptp value out of range

-2  :     Slope value out of range

-3  :     Auto value out of range

-4  :     Source value out of range

-5  :     Coupling value out of range

-6  :     Vertical Mode = XY

230  :     Error writing to instrument

232  :     Instrument not initialized

Others  :     See rs232err values